North Fork Fire District

Wildland-Urban Interface Quick Reference Guide

Gates and Codes*(Code 403.1)

Key box: will be installed in an accessible location and gate must be

minimum of 12-feet wide.

Roads, Bridges, Surfaces and Signs*(Code 403.1, 403.3 & 403.4)

Access Road: Fire apparatus access roads shall be all-weather roads with a

minimum width of 20 feet and a clear height of 13 feet 6

inches; shall be designed to accommodate the loads and

turning radii for fire apparatus.

Vehicle Load Limits: shall be posted at both entrances to

bridges on driveways and private roads.

All-Weather Road Surface: shall be any surface material acceptable to the

code official that would normally allow the passage of

emergency service vehicles to protect structures and wildlands

within the jurisdiction.

Marking of roads: Approved signs or other approved notices shall be

provided and maintained for access roads and driveways to

identify such roads and prohibit the obstruction thereof or

both.

Driveways*(Code 403.2)

Driveways: shall provide a minimum unobstructed width of 12 feet and a

minimum unobstructed height of 13 feet 6 inches

Turn-a-rounds: Driveway turnarounds shall have inside turning radii of

not less than 30 feet and outside turning radii of not less

than 45 feet.

All-Weather Road Surface: shall be any surface material acceptable to the

code official that would normally allow the passage of

emergency service vehicles to protect structures and wildlands

within the jurisdiction.

Address Marking *(Code 403.6)

Address Markers: All buildings shall have a permanently posted

address, which shall be placed at each driveway entrance and

be visible from both directions of travel along the road.

Action Items to Improve Your Home’s Survivability:

- REMOVE leaves, pine needles, and flammable

material from the roof, gutters, and on and under the

deck to help prevent embers from igniting your home.

- SCREEN areas below decks and porches with 1/8”

wire mesh to help prevent material from accumulating.

underneath.

- COVER exterior attic and soffit vents with 1/8” wire mesh

to help prevent sparks from entering your home.

- ENCLOSE eaves to help prevent ember entry.

- INSPECT shingles or roof tiles.

- REPLACE missing shingles.

Tips for Landscaping Around Your Home

- REMOVE dead vegetation and other flammable

materials, especially within the first 5 feet of the home.

- KEEP your lawn hydrated and maintained. If it is brown,

cut it down to help reduce fire intensity.

- PRUNE tree limbs so the lowest branches are 6 to 10

feet above the ground to help reduce the chance of fire

getting into the crowns of the trees.

- MOVE construction material, trash, and woodpiles

at least 30 feet away from the home and other

outbuildings.

- DISPOSE of branches, weeds, leaves, pine needles, and

grass clippings that you have cut to reduce fuel for fire.

*2006 UTAH WILDLAND- URBAN INTERFACE CODE

The information posted is meant to be a point of reference only, if more detailed code information is need, please refer to the code book.