13.60.010 Minimum Improvement Requirements

G. Sidewalks. Sidewalks shall be provided in all subdivisions on both sides of the street. Sidewalks shall be a minimum of four (4') feet wide and four (4") inches thick except  as otherwise specified in the Master Trails Plan. Sidewalks shall be installed by the developer as part of the required development improvements, not by the homeowner as part of their building permit.

H. Curb and Gutter. All streets within the City shall be improved with pavements bounded by integral concrete with high-back curb and sidewalks unless otherwise approved. All curbs shall tie into the storm water plans for water retention on-site with retention ponds and shoulder work as described in this section of this Ordinance. The contractor shall install curbs, gutters and sidewalks on streets housing public facilities, such as US Post Oﬃce, churches, etc., in the residential zones. Curbs and gutters shall be concrete of the standard high-back type unit, not less than two feet (2’) in overall width, and not less than six (6") inches thick, where the curb abuts the street pavement. Minimum grades for curb and gutter shall not be less than ½ percent. Curb cuts for residential driveways will be limited to 30' maximum. Horseshoe driveways on corner lots that access two (2) diﬀerent streets are prohibited. The required road width will be measured from the outside edge of the concrete curb which will be designated the top-back-of-curb as shown on road details for road widths in the Road Master Plan. The homeowner will be responsible for the maintenance of the gravel or grassy swale where they exist as part of the stormwater system. In the event that the swale shall lose the functionality to handle stormwater as designed, the homeowner will be responsible to rebuild the swale.