Ordinance Concept – Private Lanes and Shared Driveways

Section X.XX – Shared Driveways and Private Lanes

A. Purpose
The purpose of this section is to clarify when multiple dwellings may share a common access and to establish minimum standards for safety, maintenance, and long-term use. This ensures adequate access for residents, service vehicles, and emergency responders, and prevents the creation of substandard quasi-streets.

B. Definitions

- Driveway: A private access that serves only one dwelling unit or lot.

- Shared Driveway: A private access serving up to two dwelling units or lots, with a recorded cross-access easement.

- Private Lane: A privately owned and maintained access that serves more than two and not more than four dwelling units or lots, built to standards established in this section.

- Street: Any access intended to serve more than four dwelling units or lots shall be constructed and dedicated as a public street in accordance with City standards.

C. Standards

- Driveways:

  - Minimum width: 12 feet.

  - May serve only one dwelling unit or lot.

- Shared Driveways (≤ 2 dwellings):

  - Minimum width: 16 feet, all-weather surface.

  - Recorded access and maintenance easement required.

  - Maximum length: 150 feet unless an approved turnaround is provided.

- Private Lanes (3–4 dwellings):

  - Minimum paved width: 20 feet (24–26 feet if required by Fire Code).

  - Maximum length: 400 feet without an approved turnaround.

  - Must provide a hammerhead or cul-de-sac meeting Fire Marshal standards if exceeding 150 feet.

  - Utilities must be located within recorded easements of adequate width.

  - A recorded perpetual access and maintenance agreement among all served lots is required.

  - No parking permitted within the lane except in designated pull-outs approved by the City.

- Streets (5+ dwellings):

  - Any proposal to serve five or more dwellings or lots must dedicate a public street built to City standards.

D. General Provisions

- Emergency Access: All shared driveways and private lanes must comply with the International Fire Code (IFC) as adopted by the City.

- Trash Collection: Where private lanes cannot accommodate collection vehicles, the City may require a designated shared collection point along a public street.

- Ownership & Maintenance: The City shall not accept responsibility for the construction, maintenance, snow removal, or repair of any shared driveway or private lane.

- Subdivision Review: The creation of more than two lots accessed by a shared driveway or private lane shall require subdivision approval.