ANNEXATION PARCEL DESCRIPTION

A parcel of land, situate in the Northeast Quarter of Section 1, Township 28 South, Range 2 East, and in the Southeast Quarter of Section 36, Township 27 South, Range 2 East, Salt Lake Base and Meridian, to be annexed into the Town of Loa, more particularly described as follows:

Beginning at the existing northeast corner of the boundary of the Town of Loa, which is located North 89°17’54” West 1132.24 feet along the measured Section line and South 0°07’18” West 74.23 feet from the found monument at the Northeast Corner of Section 1, Township 28 South, Range 2 East, Salt Lake Base and Meridian, and running:

thence North 89°52’12” West 1460.12 feet, along the existing north boundary of the Town of Loa, to its intersection with the east right-of-way line of Highway 24;

thence Northwesterly along said east right-of-way line, being a spiral curve, approximated by the following described simple curve: Northwesterly 24.18 feet along the arc of a 2437.80-foot radius non-tangent curve to the left (center bears South 82°55’21” West, and the long chord bears North 7°21’42” West 24.18 feet, through a central angle of 0°34’06”);

thence Northwesterly 66.61 feet along the arc of an 869.00-foot radius non-tangent compound curve to the left (center bears South 79°03’32” West, and the long chord bears North 13°08’13” West 66.59 feet, through a central angle of 4°23’30”), along said east right-of-way line, to its intersection with the Section (Township) line;

thence South 89°45’28” East 23.56 feet along the Section line, to the calculated position of the North Quarter Corner of said Section 1 (based on record measurements shown on the old right-of-way maps);

thence South 89°52’14” East 508.57 feet along an existing wire and chainlink fence line, (possibly constructed along the original Section line), to a corner thereof;

thence South 5.08 feet along the west line of 100 East Street, to the Section line;

thence South 89°17’54” East 179.61 feet along the Section line, to the west boundary of the “Sleeping Legends Ranch Subdivision”, as recorded in the Wayne County Recorder’s office;

thence South 0°12’19” West 18.26 feet along said west boundary, to the southwest corner of said subdivision, lying on the north right-of-wat line of Highway U-72;

thence Northeasterly 533.11 feet along the arc of a 1942.90-foot radius non-tangent curve to the right (center bears South 15°09’14” East, and the long chord bears North 82°42’24” East 531.44 feet, through a central angle of 15°43’17”), along said north right-of-way line and the south boundary of said subdivision;

thence South 0°34’03” West 7.00 feet along said right-of-way line;

thence South 89°25’57” East 239.88 feet along said right-of-way line;

thence South 0°07’18” West 123.44 feet, to the Point of Beginning.

Parcel contains: 149,076 square feet, or 3.42 acres.