Sidewalk Management, Maintenance & Inspection Program (SMMIP)

Purpose

The purpose of the Sidewalk Management, Maintenance, and Inspection Program (SMMIP) is to establish a proactive, systematic, and defensible approach to ensuring sidewalks are safe, accessible, and compliant with applicable standards. This program reduces liability exposure, improves public safety, supports ADA compliance, and extends the life of sidewalk infrastructure.

Program Objectives

The SMMIP is designed to:

- Identify and correct sidewalk hazards before they cause injury

- Maintain compliance with ADA, PROWAG, and local accessibility requirements

- Reduce slip, trip, and fall claims and associated legal exposure

- Prioritize maintenance funding using risk-based decision making

- Create defensible inspection records and repair histories

- Improve walkability, mobility, and public confidence

Program Scope

This program applies to all sidewalks, curb ramps, pedestrian pathways, crosswalks, and pedestrian-related concrete infrastructure owned, operated, or maintained by the organization, including:

- Public rights-of-way

- Parks, campuses, and public facilities

- Parking lots and internal pedestrian routes

- Trails and shared-use paths

Program Components

1. Asset Inventory

All sidewalks and pedestrian assets should be mapped and inventoried using GIS or an equivalent asset management system. Each segment should include:

- Location

- Length and width

- Surface type

- Age (if known)

- Adjacent land use

- ADA features (ramps, landings, detectable warnings)

2. Inspection Program

Sidewalks will be inspected on a defined cycle based on risk level:

Area Type							Inspection Frequency

Downtown, schools, transit, high pedestrian volume	1 times per year

Residential Neighborhoods					Every 2-3 years

Low-use or rural areas					Every 3-5 years

After major storms or construction				As needed

Inspections will document:

- Vertical displacement (lips, cracks, heaving)

- Surface defects (spalling, potholes, broken panels)

- ADA compliance (cross slope, running slope, ramp condition)

- Obstructions (vegetation, poles, utility conflicts)

- Drainage and ponding

- Utility access covers, water meter and manhole covers

3. Condition Rating System

Each sidewalk segment will be assigned a condition and risk score based on:

- Size of defect

- Trip hazard severity

- Pedestrian exposure

- ADA impact

- History of complaints or claims

Ratings will fall into:

- Priority 4 :    <.25” of variation      - No action required

- Priority 3 :    .25-1” of variation     - Monitor and schedule repair if between .5”-1”

- Priority 2 :    1-2” of variation         - Repair required

- Priority 1 :    >2” of variation           - Immediate hazard; temporary mitigation required

4. Maintenance & Repair Standards

Repairs will follow established engineering and accessibility standards including:

- ADA and PROWAG requirements

- Local sidewalk design standards

Approved repair methods may include:

- Panel replacement

- Grinding

- Asphalt or concrete patching

- Mud-jacking or slab leveling

- Curb ramp upgrades

5. Prioritization Framework

Projects will be prioritized using a risk-based model that considers:

- Safety risk (Priority 1 followed by Priority 2)

  - Within 72 hours:

    - Clearly mark with safety orange paint or close and barricade section.

    - Schedule for immediate repair/replacement as soon as possible

- ADA non-compliance

- Pedestrian volume

- Proximity to schools, transit, and public buildings

- Cost vs. benefit

This ensures funding is spent where it reduces the greatest risk and improves accessibility the most.

6. Work Order & Tracking System

All hazards, inspections, and repairs will be tracked in a centralized system that records:

- Date of inspection

- Inspector name

- Condition ratings

- Photos

- Repair type

- Date of completion

- Contractor or crew

This creates a defensible record for claims, audits, and ADA reviews.

7. Public Reporting & Response

A reporting system will allow the public to submit sidewalk concerns. All complaints will be:

- Logged

- Inspected within 24 hours

- Assigned a risk level

- Tracked through resolution

8. Legal & Risk Management Integration

The SMMIP creates documented proof of reasonable care by demonstrating:

- Routine inspections

- Timely response to hazards

- Consistent repair standards

- ADA compliance efforts

This reduces exposure in personal injury and accessibility lawsuits.

Performance Metrics

The program will track:

- Miles of sidewalk inspected annually

- Number of hazards identified

- Percentage of high-risk defects repaired

- Average response time

- Reduction in claims and complaints

- ADA compliance improvements

Conclusion

A formal Sidewalk Management, Maintenance, and Inspection Program allows the organization to move from reactive repairs to a proactive, data-driven system that protects the public, controls costs, improves accessibility, and reduces legal risk.

Sidewalk and Gutter Defects

Sidewalk and gutter defects refer to any sidewalk or gutter anomaly causing or potentially causing a safety risk or functionality issue. Defects are identified through inspections as well as individuals informing the organization of the issue. Each defect is inspected and the information is recorded. The Defect Rating System is as follows:

- LOW - Defect is visible.

- MODERATE - Defect is a potential hazard.

- HIGH - Defect is a safety hazard or non-functional.

- EXTREME - Defect is hazardous. Priority replacement.

Defect descriptions are divided into several categories.

- Cracking and Holes: (Causes: Compaction issues, heavy drive loads, expansion)

  - Low - Crack or hole is visible but not causing unsafe conditions. (Hairline cracks)

  - Moderate - 1/4” - 1/2” separation, lift, settle or chipping.

  - High - Between 1/2” and 1” separation, lift, settle or chipping.

  - Extreme - Greater than 1” separation, lift, settle or chipping.

High - Cracking

- Settling and Heaving: (Causes: Trees, utilities, poor compaction)

  - Low - < 1/2”.

  - Moderate - 1/2” to 1”.

  - High - >1” to 2”.

  - Extreme - > than 2”.

Extreme - Heaving

- Spalling: (Causes: Weak surface, freeze/thaw cycles, poor mix design or over finishing)

  - Low - Minimal shallow, random flaking.

  - Moderate - Up to 25% of the slab is spalling. Exposed aggregate, 1/2” recesses.

  - High - Up to 50% of the slab is spalling. Exposed aggregate, >1/2” recesses.

High - Spalling

Extreme - Over 50% of the slab is spalling. Exposed aggregate, > 3/4” recesses.

- Drainage: (Causes: Settling, improper grade)

  - Low - Small area puddle.

  - Moderate - Water puddles to the road surface.

  - High - Water puddles onto the road or approach causing damage or ice buildup or water backup.

  - Extreme-Water puddles onto the road or sidewalk causing damage, flooding, or ice buildup.

High - Drainage

Area Type coverages  as identified on GIS Sidewalk Map