Title 5 Stormwater

5.04 General Information

5.08 Land Disturbance Permits

5.12 Stormwater System Design And Management Standards

5.16 Post Construction

5.20 Waivers

5.24 Existing Locations And Developments

5.28 Inspections

5.32 Illicit Discharges

5.36 Enforcement

5.40 Penalties

5.44 Appeals

# HISTORY

## Amended by Ord. 11-15-2023-A on 11/15/2023

5.04 General Information

5.04.010 Findings

5.04.020 General Provisions

5.04.030 Definitions: Stormwater Ordinance

5.04.040 Stormwater Utility

5.04.050 Stormwater Utility Fee

5.04.060 Billing

### 5.04.010 Findings

The City Council makes the following findings regarding stormwater runoff and the city’s stormwater management facilities system:

- Historically, the city’s stormwater system consisted of a network of ditches that were previously used for irrigation. Those ditches are being removed as improvements are made by installing curb & gutter. With the elimination of the ditches, the stormwater cannot be dispersed.

- The city’s existing culverts and remaining ditches do not adequately handle the stormwater runoff generated in the City.

- The city’s anticipated growth will place increased demands on the already inadequate stormwater system.

- Uncontrolled or inadequately controlled stormwater runoff causes erosion and property damage.

- Uncontrolled or inadequately controlled stormwater runoff hinders the city’s ability to provide emergency services to its residents.

- Uncontrolled or inadequately controlled stormwater runoff impedes the regular flow of traffic in the City.

- Uncontrolled or inadequately controlled stormwater runoff poses hazards to the citizens of the community.

- Stormwater runoff carries concentrations of oil, grease, nutrients, chemicals, heavy metals, toxic materials, and other undesirable materials that may jeopardize the integrity of ground waters and receiving waters including the city’s culinary water supply.

- All developed properties in the City contribute to the need for the stormwater system by converting natural ground cover into impervious surfaces.

- All developed properties in the City make use of or benefit from the city’s operation and maintenance of the stormwater system.

- The State Department of Environmental Quality (DEQ) has determined that some of the city’s stormwater sumps must be included on the prioritized contamination sources for culinary wells.

- Inadequate drainage along existing streets due to the lack of an adequate stormwater system causes the street pavement to deteriorate and fail thus increasing street maintenance costs.

- Absent effective maintenance, operation, regulation, and control of existing stormwater drainage conditions in the City constitute a potential hazard to the health, safety, and general welfare of the City, its residents, and its businesses.

- A stormwater utility is the most equitable and efficient method of managing stormwater in the City and ensuring that each property in the City pays its fair share of the amount that the property contributes to, benefits from, and otherwise uses the stormwater system.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.04.020 General Provisions

Purpose. It is the purpose of this Title to:

- Protect, maintain, and enhance the environment of Payson City.

- Establish responsibilities for controlling and managing storm water runoff.

- Protect the public health, safety, and the general welfare of the citizens of the city, by controlling discharges of pollutants to the city’s storm water system and to maintain and improve the quality of the receiving waters into which the storm water outfalls flow, including, without limitation, lakes, rivers, streams, ponds, wetlands, and groundwater of the city.

- Enable the City to comply with the National/Utah Pollution Discharge Elimination System permit (NPDES /UPDES) and applicable regulations, 40 CFR §122.26 for storm water discharges.

- Allow the City to exercise the powers granted by Utah Code, which provides that, among other powers municipalities have with respect to stormwater facilities, is the power by ordinance or resolution to:

  - Exercise general regulation over the planning, location, construction, and operation and maintenance of stormwater facilities in the municipality, whether owned and operated by the municipality or not;

  - Adopt any rules and regulations deemed necessary to accomplish the purposes of this statute, including the adoption of a system of fees for services and permits;

  - Establish standards to regulate the quantity of stormwater discharged and to regulate stormwater as may be necessary to protect water quality;

  - Review and approve plans and plats for stormwater management in proposed subdivisions or commercial developments;

  - Issue permits for stormwater discharges, or for the construction, alteration, extension, or repair of stormwater controls;

  - Suspend or revoke permits when it is determined that the permittee has violated any applicable ordinance, resolution, or condition of the permit;

  - Regulate and prohibit discharges into stormwater facilities of sanitary, industrial, or commercial sewage or waters that have otherwise been contaminated; and

  - Expend funds to remediate or mitigate the detrimental effects of contaminated land or other sources of stormwater contamination, whether public or private..

- Provide annual training for all employees, contracted staff, and other responsible entities that have the primary operation or maintenance job functions likely to impact stormwater quality covering all required training addressed in the SWMP.

- Provide training for new hires that have the primary operation or maintenance job functions likely to impact stormwater quality within 60 days of hire covering all required training addressed in the SWMP.

- State Law Compliance. The City shall administer, interpret, and enforce this Title in accordance with Utah Code §19-5-108.3.

- Limitation on Regulatory Authority. The City shall not adopt, implement, or enforce any requirement for construction stormwater runoff control that is more stringent than the minimum requirements of the federal Clean Water Act., unless expressly authorized by state statute.

The City Engineer shall administer the provisions of this Title. Nothing in this Title shall relieve any person from responsibility for damage to other persons or property, nor impose upon Payson City, its officers, agents, or employees, any liability for damage to other persons or property.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.04.030 Definitions: Stormwater Ordinance

For the purpose of this Title, the following definitions shall apply. Words used in the singular shall include the plural, and the plural shall include the singular; words used in the present tense shall include the future tense. The word "shall" is mandatory and not discretionary. The word "may" is permissive. Words not defined in this Section shall be construed to have the meaning given by common and ordinary use as defined in the latest edition of Webster's Dictionary.

APPLICANT means a person applying for a construction stormwater permit

AS BUILT PLANS means drawings depicting conditions as actually constructed.

BEST MANAGEMENT PRACTICES (BMPs)  means methods, measure, or practices consistent with the Federal Clean Water Act and applicable federal regulations. The City shall maintain and publish a list of preferred BMPs on its official website.

CONSTRUCTION STORMWATER PERMIT means a permit required for construction activity distiurbing one acre or more, or part of a common plan.

CHANNEL means a natural or artificial watercourse with a definite bed and banks that conducts flowing water continuously or periodically.

CITY ENGINEER means the City Engineer of Payson City or an authorized designee.

COMMUNITY WATER means any and all rivers, streams, creeks, branches, lakes, reservoirs, ponds, drainage systems, springs, wetlands, wells, and other bodies of surface or subsurface water, natural or artificial, lying within or forming a part of the boundaries of the City.

CONTAMINANT means any physical, chemical, biological, or radiological substance or matter that is not naturally occurring and introduced into the stormwater system.

DESIGN STORM EVENT means a hypothetical storm event, of a given frequency interval and duration, used in the analysis and design of a stormwater system.

DISCHARGE means dispose, deposit, spill, pour, inject, seep, dump, leak or place by any means, or that is disposed, deposited, spilled, poured, injected, seeped, dumped, leaked, or placed by any means including any direct or indirect entry of any solid or liquid matter into the municipal separate stormwater system.

EASEMENT means an acquired privilege or right of use or enjoyment that a person, party, firm, corporation, municipality, or other legal entity has in the land of another.

ELECTRONIC SITE INSPECTION means geo-located and time-stamped photographs submitted electronically to demonstrate compliance.

EROSION means the removal of soil particles by the action of water, wind, ice, or other geological agents, whether naturally occurring or acting in conjunction with or promoted by anthropogenic activities or effects.

EROSION AND SEDIMENT CONTROL PLAN means a written plan (including drawings or other graphic representations) that is designed to minimize the accelerated erosion and sediment runoff at a site during construction activities.

GENERAL CONSTRUCTION STORMWATER PERMIT means a permit required by the Utah Department of Environmental Quality, Division of Water Quality.

HOT SPOT (PRIORITY AREA) means an area where land use or activities generate highly contaminated runoff, with concentrations of pollutants in excess of those typically found in stormwater.

ILLICIT CONNECTIONS means illegal and/or unauthorized connections to the municipal separate stormwater system whether or not such connections result in discharges into that system. Any drain or conveyance whether on the surface or subsurface, which allows a contaminated or illicit discharge to enter the stormwater system. Examples include, but are not limited to, any conveyances that allow non-stormwater discharge such as sewage, process wastewater, or wash water to enter the stormwater system, and any connections to the stormwater system from indoor drains or sinks regardless of whether said drain or connection had been previously allowed, permitted, or approved by a government agency; or any drain or conveyance connected to or discharging to the stormwater system, which has not been (1) documented in plans, maps, or equivalent records submitted to the City, and (2) approved in writing by the City.

ILLICIT DISCHARGE means any discharge to the municipal separate stormwater system that is not composed entirely of stormwater and not specifically exempted under this Title. Illicit discharges include both direct connections (e.g., wastewater piping either mistakenly or deliberately connected to the stormwater system) and indirect connections (e.g., infiltration into the stormwater system or spills collected by drain inlets).

IRRIGATION DITCHES means gravity irrigation ditches used by irrigation shareowners having a right of water passageway by ROW, easement, or prescription. Irrigation ditches also include those facilities that function as a combined stormwater and irrigation conveyance intended at times as a water routing and disposal system.

LAND DISTRIBUTION PERMIT means a Payson City Land Disturbance Permit as adopted by the City.

LAND DISTRIBUTING ACTIVITY means any activity on property that results in a change in the existing soil cover (both vegetative and non-vegetative) and/or the existing soil topography. Land-disturbing activities include, but are not limited to, development, re-development, demolition, construction, reconstruction, clearing, grading, filling, and excavation.

MAINTENANCE means any activity that is necessary to keep a stormwater system in good working order so as to function as designed. Maintenance shall include complete reconstruction of a stormwater system if reconstruction is needed in order to restore the facility to its original operational design parameters. Maintenance shall also include the correction of any problem on the site property that may directly impair the functions of the stormwater system or cause degradation of the stormwater quality.

MAINTENANCE AGREEMENT means a document recorded in the Utah County land records that acts as a property deed restriction and that provides for long-term maintenance of stormwater management practices.

MUNICIPAL SEPARATE STORM SEWER SYSTEM (MS4) means the conveyances owned or operated by the City for the collection and transportation of stormwater, including the roads and streets and their drainage systems, catch basins, curbs, gutters, ditches, man-made channels, and storm drains.

NATIONAL POLLUTANT DISCHARGE ELIMINATION SYSTEM PERMIT or NPDES PERMIT means a permit issued pursuant to 33 U.S.C. 1342.

NOTICE OF VIOLATION (N.O.V.). Whenever the City Engineer finds that a person is in non-compliance with this Title, the City Engineer will order compliance by written notice of violation to the responsible person. Requirements in this Notice are at the discretion of the City Engineer, and may include monitoring, fines, payment to cover costs relating to the non-compliance, and the implementation of Best Management Practices.

OFF-SITE FACILITY means any facility being a structural BMP or otherwise located outside the subject property boundary described in the permit application for land development activity that is intended to form an integral part of the stormwater system for a given parcel.

ON-SITE FACILITY means a structural BMP located within the subject property boundary described in the permit application for land development activity.

OVERSIGHT INSPECTION means an inspection conducted by the City to assess compliance.

PEAK FLOW means the maximum instantaneous rate of flow of water at a particular point resulting from a storm event.

PERSON means any individual, corporation, partnership, association, company, or body politic, including any agency of the State of Utah and the United States government, which includes both the singular and plural form of said groups, who acts to discharge to or otherwise influence the stormwater system in the City.

PRE-EXISTING CONDITIONS means conditions of property in its native state or changed under approval by the City or changed property that is grandfathered.

PRIORITY AREA means HOT SPOT as defined in this Section.

PROPERTY OWNER means a landowner of property within the boundary of the City.

RUNOFF means that portion of the precipitation on a drainage area that is discharged from the area into the municipal separate stormwater system. Also, water produced by storms, surface drainage, snow and ice melt, and other water handled by the stormwater system.

SEDIMENT means solid material, both mineral and organic, that is in suspension, is being transported, or has been moved from its site of origin by air, water, gravity, or ice and has come to rest on the earth's surface either above or below sea level or within any stormwater system.

SEDIMENTATION means soil particles that can settle or have settled in the stream beds and disrupt the natural flow of the stream or otherwise disrupt the intended stormwater system function.

SOILS REPORT means a study of soils on a subject property with the primary purpose of characterizing and describing the soils. The soils report shall be prepared by a qualified soils engineer, who shall be directly involved in the soil characterization either by performing the investigation or by directly supervising employees.

STABILIZATION means providing adequate measures, vegetative and/or structural, that will prevent erosion from occurring.

STORMWATER means stormwater runoff, snow melt runoff, surface runoff, street wash waters related to street cleaning or maintenance, infiltration, and drainage.

STORMWATER DESIGN STANDARDS AND REGULATIONS means the current Payson City stormwater standards and regulations as adopted by the City whether in this Title or the Payson City Development Guidelines.

STORMWATER MASTER PLAN means the current Payson City Stormwater Master Plan as adopted by the City.

STORMWATER MANAGEMENT means the programs to maintain quality and quantity of stormwater runoff to pre-development levels.

STORMWATER SYSTEM means city-owned systems including without limitation the drainage structures, conduits, ditches, combined sewers, sewers, gutters, catch basins, grates, pipelines, and property encumbrances that receive runoff from public right-of-way, private property, natural waterways, and systems identified as city easements including all device appurtenances by means of which stormwater is collected, transported, pumped, treated, or disposed of.

STORMWATER POLLUTION PREVENTION PLAN (SWPPP) means the set of drawings and other documents showing the location of the BMPs during the different phases of construction and system management and that comprise all the information and specifications for the programs, drainage systems, structures, BMPs, concepts, and techniques intended to maintain or restore quality and quantity of stormwater runoff to pre-development levels during and after construction.

STORMWATER MANAGEMENT PROGRAM or SWMP means a Technical Report including a copy of the Land Disturbance Permit, Notice of Intent (NOI) (if applicable), Stormwater Pollution Prevention Plan, during construction and post construction, stormwater pollution prevention BMPs, spill prevention and countermeasure information, inspection records, and signed and dated Certification Statement from the Site Operator and the responsible person preparing the report.

STORMWATER RUNOFF means flow on the surface of the ground or within the stormwater system, resulting from precipitation or other forms of water origination being those that are both natural and artificially occurring.

STORMWATER UTILITY means the stormwater department created by ordinance of the City to administer this Title 5 Stormwater, and other stormwater rules and regulations adopted by the City to maintain and manage the stormwater system of the City.

STRUCTURAL BMPS means devices that are constructed to provide control of both the quality and quantity of stormwater runoff.

SURFACE WATER means and includes waters upon the surface of the earth in bounds created naturally or artificially including, but not limited to, streams, other water courses, lakes, reservoirs, ponds, sloughs, or other bodies of water.

WATERCOURSE means a permanent or intermittent stream or other body of water, either natural or man-made that gathers or carries surface water.

WATERSHED means all the land area that contributes runoff to a particular point along a waterway or other stormwater conveyance.

UPDES means Utah Pollution Discharge Elimination System.

- Property owners proposing to redirect runoff, surface, and/or pipe flow to properties or facilities outside Payson City boundaries must provide written approval from the state, county, or municipality or their agents.

- Property owners are responsible for the protection of canals per the relevant sections of this Title.

- Discharges or modifications to the canals require written approval from the canal owners and applicable governing agencies.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.04.040 Stormwater Utility

- Creation. The City Council hereby creates and establishes a stormwater utility as part of the City’s overall sewer system. The stormwater utility shall plan, design, construct, maintain, administer, and operate the city’s stormwater

system.

- Enterprise Fund. The City Council hereby establishes a stormwater utility enterprise fund to handle all income, expenses, and other financial transactions related to the stormwater utility. All stormwater utility service charges shall be deposited in the stormwater utility enterprise fund. Money in the stormwater utility enterprise fund shall not be commingled with or transferred to other city funds. However, the stormwater utility may pay other city funds for services and expenses directly attributable to the stormwater utility. The stormwater utility enterprise fund shall be operated according to state law and city policy.

- Facilities And Assets. The stormwater utility shall operate independently of city operations funded by the general fund. The stormwater utility shall have the same relationship to the City as other city utilities, such as the water utility and the sanitary sewer and wastewater utility. Upon creation of the stormwater utility, all of the city’s stormwater system and assets (other than streets and other facilities and assets designated by the City Manager) shall be transferred to the stormwater utility in consideration for the stormwater utility’s agreement to take primary responsibility for planning, designing, constructing, maintaining, administering, and operating the city’s stormwater.

- Administration. The stormwater utility shall be administered by the city’s Street and Stormwater Superintendent.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.04.050 Stormwater Utility Fee

- Imposed. Each developed parcel of real property in the City shall be charged a stormwater utility fee. However, when the City annexes property into the municipal limits of Payson City, no stormwater utility fee shall be assessed on the annexed property until such time as the annexed property is developed, until a stormwater system is installed, or as directed by an annexation agreement.

- ESU. The stormwater utility fee shall be based on the number of equivalent service units (ESU’s) contained in the parcel. The City Council finds that the ESU is the most accurate measurement for determining the amount that each parcel contributes to, benefits from, and otherwise uses the stormwater system. Based on a study completed by an independent engineer, the City Council finds and establishes that one ESU equals 2,700 square feet of impervious surface area.

- Calculation. The City Council finds that each single-family residential parcel contributes approximately the same amount of stormwater runoff; therefore, each developed single-family residential parcel shall pay a base rate of one

(1) ESU. All non-single-family residential parcels shall pay a multiple of this base rate, expressed in ESU’s according to the measured impervious area on the parcel. The City Council may adopt separate rates for PRD’s, condominiums, and other uses that are not easily handled under the standard rate schedule.

- Charge Per ESU. The amount charged for each ESU shall be set forth in the City Fee Schedule.

- Exemptions And Credits. The City Council may establish exemptions and credits to the stormwater utility fee as set forth in the City Fee Schedule.

- Policies. The Street and Stormwater Superintendent may adopt policies, consistent with this Title and any resolutions passed by the City Council, to assist in the application, administration, and interpretation of this Title and any resolutions related to the stormwater utility.

- Appeals. Any person or entity that believes that this Title, or any stormwater utility fee, was interpreted or applied erroneously may appeal to the Street and Stormwater Superintendent (“Superintendent”). The appeal shall be in writing, shall state any facts supporting the appeal, and shall be made within ten (10) days of the decision, action, or bill being appealed. The Superintendent may elect to hold a hearing on the appeal. The Superintendent shall decide the appeal within ten (10) days of when the appeal is filed. If the person or entity is not satisfied with the Superintendent’s decision, a further appeal may be made to the City Manager (or designee). The appeal to the City Manager shall follow the same procedure as the appeal to the Superintendent. The City Manager’s decision shall be final and binding on all parties.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.04.060 Billing

The City Council finds that the city’s stormwater system, sanitary sewer and wastewater system, culinary water system, and solid waste collection system are interrelated services that are part of a unified City plan to provide for the health, safety, and welfare of the City and its residents in an environmentally responsible manner. Therefore, the stormwater utility fee shall be included on the city’s regular monthly utility bill for any given property. The stormwater utility fee shall be charged to the owner of the property. The fee shall be deemed a civil debt owed to the City by the person or entity paying for the city utility services provided to the property. All properties shall be charged the stormwater utility fee, regardless of whether or not the owner or occupant of the property requests the stormwater utility service. Failure to pay any portion of the utility bill may result in termination of water service.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

5.08 Land Disturbance Permits

5.08.010 When Required

5.08.020 Drainage Channels, Waterways, And Sensitive Areas

5.08.030 Building Permit

5.08.040 Exemptions

5.08.050 Application For A Land Disturbance Permit.

5.08.060 Review And Approval Of Application

### 5.08.010 When Required

Every person will be required to obtain a land disturbance permit from the City Engineer in the following cases:

- Land disturbing activity that generally disturbs one (1) or more acres of land.

- Land disturbing activity of less than one (1) acre of land if such activity is part of a larger common plan of development that affects one (1) or more acre of land.

- Land disturbing activity of less than one (1) acre of land, if in the discretion of the City Engineer such activity poses a unique threat to water, public health, or safety.

- The creation and use of borrow pits.

- Development of a single-family home.

- Processing of earthen materials such as top soil and gravel screening.

- Construction of parking lots.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.08.020 Drainage Channels, Waterways, And Sensitive Areas

Property owners shall not alter or restrict natural channels without proper Federal, State, and City permits.

Modifications of sensitive areas are subject to and governed by PCC 14 Sensitive Lands. These actions will require a Land Disturbance Permit and approval from all other governing agencies.

Property owners proposing to redirect runoff, surface and/or pipe flow, to properties or facilities outside Payson City boundaries must provide written approval from the state, county, municipality, or their agents.

Property owners are responsible for the protection of canals per the relevant sections of this Title.

Discharges or modifications to the canals require written approval from the canal owners and applicable governing agencies.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022

Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.08.030 Building Permit

No building permit shall be issued until the applicant has obtained a Land Disturbance Permit where the same is required by this ordinance.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022

### 5.08.040 Exemptions

The following activities are exempt from the permit requirement:

- Any emergency activity that is immediately necessary for the protection of life, property, or natural resources including activities required to promote public safety, repairs to water lines, and/or other City infrastructure repairs.

- Existing nursery and agricultural operations conducted as a permitted main or accessory use.

- Any agricultural activity that is consistent with an approved farm conservation plan or a management plan prepared or approved by the appropriate City, Federal, or State Agency.

- Additions or modifications to existing single-family structures.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.08.050 Application For A Land Disturbance Permit.

- 1. Name of applicant;

  - Business or residence address of applicant;

  - Name, address, and telephone number of the owner of the property of record in the office of the Utah County Assessor;

  - Address and legal description of subject property including the tax reference number and parcel number of the subject property;

  - Name, address, and telephone number of the contractor and any subcontractor(s) who shall perform the land disturbing activity and who shall implement the erosion and sediment control plan;

  - A statement indicating the nature, extent, and purpose of the land disturbing activity including the size of the area for which the permit shall be applicable and a schedule for the starting and completion dates of the land disturbing activity.

  - The applicant shall obtain from any other state or federal agency any other appropriate environmental permits that pertain to the property such as the UPDES Stormwater General Permit for Construction Activities UTRC00000 for projects disturbing greater than or equal to one acre, or UPDES Common Plan Permit UTRH00000 for projects less than one acre that are part of a larger common plan of development or sale that will collectively disturb greater than or equal to one acre. However, the City Engineer may impose conditions necessary to ensure compliance with this Title and applicable federal and state law, provided that such conditions shall not exceed the minimum requirements of the federal Clean Water Act unless expressly authorized by state statute.

  - Each application shall be accompanied by a stormwater pollution prevention plan (SWPPP) providing for erosion and sediment control and stormwater during the land disturbing activity and after the activity has been completed meeting the requirements of the Stormwater General Permit for Construction Activities Permit No. UTRC00000 or the Common Plan Permit No. UTRH00000. Each application for a land disturbance permit shall be accompanied by payment of land disturbance permit fees as set forth in the City Fee Schedule.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.08.060 Review And Approval Of Application

The City Engineer will review each application for a Land Disturbance Permit to determine compliance with this Title. A complete application shall be defined by application requirements published and maintained by the City on its official website. The City shall review a complete Storm water Pollution Prevention Plan (SWPP) within fourteen (14) business days after determining the application is complete in accordance with Utah Code §19-5-108.3. No land disturbing activity requiring permit coverage shall commence until the SWPPP is approved or otherwise authorized by the City. If an application or SWPPP is incomplete, the City shall notify the applicant of deficiencies and the review period shall not begin until the application is complete. Within fourteen (14) business days after determining the application is complete, the City Engineer shall provide one of the following responses in writing:

- Approval of the permit application;

- Approval of the permit application, subject to such reasonable conditions as may be necessary to secure substantially the objectives of this Title, and issue the permit subject to these conditions; or

- Denial of the permit application, indicating the reason(s) for the denial.

If the City Engineer has granted conditional approval of the permit, the applicant shall submit a revised plan that conforms to the conditions established by the City Engineer. However, the applicant shall be allowed to proceed with the land disturbing activity so long as it conforms to conditions established by the City Engineer.

No development plans will be released until the land disturbance permit has been approved.

Permit Duration. Every land disturbance permit shall expire and become null and void if substantial work authorized by such permit has not commenced within one hundred eighty (180) calendar days of issuance or is not complete within eighteen (18) months from the date of the commencement of construction.

Notice Of Construction. The applicant must notify the City Engineer ten (10) working days in advance of the commencement of construction.

Inspection. Regular inspections of the stormwater system construction shall be conducted by the party responsible for the work and reviewed by the City Engineer. The property owner shall allow access to the City Engineer or a representative to inspect storm water control measures that discharge to the MS4. The inspection shall review the control measures in place, the maintenance plan, and the need for additional measures to completely address the erosion and sediment control for the project.

All inspections shall be documented and written reports prepared that contain the following information:

- The date and location of the inspection;

- Whether construction is in compliance with the approved stormwater management program;

- Variations from the approved construction specifications;

- Any violations that exist.

- Performance bonds.

The City Engineer may, at their discretion:

- Require the submittal of a performance security or performance bond prior to issuance of a permit in order to ensure that the stormwater practices are installed by the permit holder as required by the approved stormwater management program.

- The amount of the performance security or performance bond shall be the total estimated construction cost of the structural BMPs approved under the permit plus any reasonably foreseeable additional related costs, e.g., for damages or enforcement.

- The performance security or performance bond shall contain forfeiture provisions for failure to complete work specified in the stormwater management program. If the work is not started within one hundred eighty (180) days of the pre-construction meeting, the bond will be returned minus any utility modeling fees that will be taken from the testing and inspection portion of the performance security or performance bond.

If the contractor starts work on the project and leaves the site for forty-five (45) continuous days, then the performance security or performance bond shall be used to hire another contractor to complete the project.

- The applicant shall provide an itemized construction cost estimate complete with unit prices, which shall be subject to acceptance, amendment, or rejection by the City Engineer.

- Alternatively, the City Engineer shall have the right to calculate the cost of construction cost estimates.

- The performance security or performance bond shall be released in full only upon submission of as-built plans written certification by a registered professional engineer licensed to practice in the State of Utah that the structural BMP has been installed in accordance with the approved plan, signed Notice of Termination of the Construction General Permit, and other applicable provisions of this Title.

The City Engineer will make a final inspection of the structural BMP to ensure that it is in compliance with the approved plan and the provisions of this Title. Provisions for a partial pro-rata release of the performance security or performance bond based on the completion of various development stages can be made at the discretion of the City Engineer.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.12 Stormwater System Design And Management Standards

5.12.010 Prior Approval And Standard

5.12.020 Irrigation Ditches

5.12.030 Stormwater Design And BMP Manuals

5.12.040 General Performance Criteria For Storm Water Management

5.12.050 Minimum Control Requirements

5.12.060 Stormwater Pollution Prevention Plan Requirements

# HISTORY

## Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.12.010 Prior Approval And Standard

When a person is engaged in development activity that requires prior approval by the City and the issuance of a land disturbance permit, the following standards apply to the design and management of any water carrying system found on the property disturbed by that person. Prior to or during the site design process, applicants for land disturbance permits shall consult with the City Engineer to determine if they are subject to stormwater design requirements in addition to what is described in this Section.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.12.020 Irrigation Ditches

All existing irrigation ditches located on the site or straddling a site property boundary shall be piped with a sufficient size pipe and shall be coordinated with the water user and the City Engineer.

Property owners are responsible for the protection of irrigation ditches per the relevant sections of this Title.

Discharges to private ditches require written approval from the ditch owners, and the design shall comply with the terms of approvals and the Payson City Development Guidelines and the Land Disturbance Permit.

Piping of ditches and modification to the diversion boxes require documented coordination with ditch owners or representative and are required to receive written approval of ditch owners. Design and coordination requirements shall comply with the Payson City Development Guidelines and the Land Disturbance Permit.

.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.12.030 Stormwater Design And BMP Manuals

The City adopts as its stormwater design and best management practices (BMP) manuals the following publications, which are adopted as administrative guidance and implementation purposes and shall be maintained by the City.

- Payson City Development Guidelines.

- Payson City Stormwater Master Plan.

- Guidance Document for Stormwater Management (EPA Construction Site Stormwater Runoff Control) available online at https://www3.epa.gov/npdes/pubs/sw_swppp_guide.pdf.

- Allowable BMP’s.

- Low Impact Development Manual.

- During and Post construction Site Inspection SOP

These manuals include a list of acceptable BMPs and include specific design performance criteria and operation and maintenance requirements for each stormwater practice. The manuals may be updated and expanded from time to time, at the discretion of the City Council, upon the recommendation of the City Engineer, based on improvements in engineering, science, monitory, and local maintenance experience. Stormwater facilities that are designed, constructed, and maintained in accordance with these BMP criteria will be presumed to meet the minimum water quality performance standards Stormwater facilities that are designed, constructed, and maintained in accordance with these BMP criteria shall be presumed to meet minimum water quality performance standards. These manuals and BMP standards shall not impose requirements more stringent than federal Clean Water Act requirements unless expressly authorized by state statute.

.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.12.040 General Performance Criteria For Storm Water Management

Unless granted a waiver or judged by the City Engineer to be exempt, the following post construction performance criteria shall be addressed for stormwater management at all sites.

- Design of stormwater systems within city boundaries that discharge into a Payson City stormwater system requires direct supervision of a Utah registered professional engineer and shall carry the seal of the same supervising professional engineer.

- Site designs shall control the peak flow rates of stormwater discharge associated with design storms specified in this Title or Development Guidelines and reduce the generation of post-construction stormwater runoff to pre-construction levels or 0.2 cfs/acre, whichever is less, according to the Development Guidelines. These practices should seek to utilize low impact design (LID) methods including using pervious areas for stormwater treatment and to infiltrate stormwater runoff from driveways, rooftops, parking lots, and landscaped areas to the maximum extent practical to provide treatment for both water quality and quantity.

- Specific channel protection criteria shall be provided as prescribed in the Development Guidelines to protect stream channels from degradation. A Stream Alteration Permit is also required for any work performed in Dry Creek or Peteetneet Creek.

- Stormwater discharges to critical areas with sensitive resources (i.e., cold water fisheries, swimming beaches, recharge areas, water supply reservoirs) may be subject to additional performance criteria, or may need to utilize or restrict certain storm water management practices.

- Stormwater discharges from “hot spots” may require the application of specific structural BMPs and pollution prevention practices.

- Prior to or during the site design process, applicants for land disturbance permits shall consult with the City Engineer to determine if they are subject to additional stormwater design requirements.

- Calculations for determining allowable peak flows and runoff volumes as found in the Development Guidelines shall be used for sizing all stormwater facilities.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.12.050 Minimum Control Requirements

Stormwater discharge during all construction activities shall comply with the terms of the UPDES Stormwater General Permit for Construction Activities UTRC00000 in the case of a project that impacts greater than or equal to one acre or the UPDES Common Plan Permit UTRH00000 in the case of a project that impacts less than an acre that is part of a larger common plan of development or sale that will collectively disturb greater than or equal to one acre, Land Disturbance Permit, the Payson City Development Guidelines. All requirements imposed under this section shall be interpreted and applied consistent with the federal Clean Water Act requirements and Utah Code §19-5-108.3, and shall not exceed those requirements unless expressly authorized by state statute.

Stormwater designs shall meet the multi-stage storm frequency storage and runoff volume requirements as identified in the Development Guidelines, along with the operation, installation, and maintenance standards in the Development Guidelines unless the City Engineer has granted the applicant a full or partial waiver for a particular BMP under PCC 5.12.040. Any design requirements applied under this section shall be consistent with the City’s published stormwater standards and shall not be more stringent than the federal Clean Water Act requirements unless expressly authorized by statute.

Runoff rates from one lot to another may not exceed pre-existing conditions or in such a manner that may unreasonably and unnecessarily cause more harm than formerly. This standard shall be applied in a manner consistent with applicable federal stormwater requirements.

If hydrologic or topographic conditions warrant additional controls, the City Engineer may impose  additional requirements necessary to ensure compliance with applicable federal and state law; however such requirements shall not exceed the federal Clean Water Act requirements unless expressly authorized by state statute.

Soil, sediment, and debris brought onto streets and public ways must be removed by the end of the work day by machine, broom, or shovel to the satisfaction of the City Engineer. Failure to remove the sediment, soil, or debris shall be deemed a violation of this Title.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.12.060 Stormwater Pollution Prevention Plan Requirements

Property owners are responsible for managing stormwater runoff and sediment whether in conduit systems or on the surface that traverse or originate on their property, unless this responsibility is relinquished through the terms and conditions of an easement. In order to manage stormwater quality, the Property Owner must develop a stormwater pollution prevention plan and implement the plan. The stormwater pollution prevention plan shall include sufficient information to allow the City Engineer or RSW to evaluate the environmental and historical characteristics of the project site, the potential impacts of all proposed development of the site, both present and future, on the water resources, and the effectiveness and acceptability of the measures proposed for managing stormwater generated at the project site. Follow the approved and most recent Department of Environmental Qualities Construction General, Common Plan Permits and Construction Dewatering permits and templates at https://deq.utah.gov/water-quality/general-construction-storm-water-updes-permits. No work can begin until Land Disturbance Fees have been paid and your SWPPP and NOI must be reviewed and approved with all BMP's in place according to your CGP/CCP SWPPP document, followed up with a SWPPP Preconstruction and training before work can begin.

To accomplish this goal, the stormwater pollution prevention plan shall include the following:

- Project Description: Briefly describe the intended project and proposed land disturbing activity, number of units, and structures to be constructed, and infrastructure required.

- Topographic Base Map: A topographic base map of the site, which extends a minimum of 1,000 feet beyond the limits of the proposed development and indicates:

  - Existing surface water drainage including streams, ponds, culverts, ditches, sink holes, wetlands; and the type, size, elevation, etc., of the nearest upstream and downstream drainage structures;

  - Current land use including all existing structures, locations of utilities, roads, and easements;

  - All other existing significant natural and artificial features including a general description of existing land cover. Individual trees and shrubs do not need to be identified.

  - Proposed land use with tabulation of the percentage of surface area to be adapted to various uses, drainage patterns, locations of utilities, roads and easements, and the limits of clearing and grading;

  - Proposed structural BMPs;

  - A written description of the site plan and justification of proposed changes in natural conditions may also be required.

Calculations. Hydrologic and hydraulic design calculations for the pre-development and post-development conditions for the design storms specified in the Development Guidelines. These calculations must show that the proposed stormwater management measures are capable of controlling runoff from the site in compliance with this Title and the specifications of the Development Guidelines. Such calculations shall include:

- A description of the design storm frequency, duration, and intensity where applicable;

- Time of concentration;

- Soil curve numbers or runoff coefficients including assumed soil moisture conditions;

- Peak runoff rates and total runoff volumes for each watershed area;

- Infiltration rates for any storage area;

- Culvert, stormwater system, ditch, and/or other stormwater capacities;

- Flow velocities;

- Data on the increase in rate and volume of runoff for the design storms referenced in the Development Guidelines; and

- Documentation of sources for all computation methods and field test results.

Soils Information. If a stormwater management control measure depends on the hydrologic properties of soils (e.g., infiltration basins, ponds, sumps, etc.), then a soils report shall be submitted. The soils report shall be based on on-site boring logs or soil pit profiles and soil survey reports. The number and location of required soil borings or soil pits shall be determined based on what is needed to determine the suitability and distribution of soil types present at the location of the control measure. In all cases where subsurface infiltration is a component of the stormwater management program, a site specific percolation test shall be submitted based upon field observations and testing at the location of the infiltration facility.

Work Sequence. The projected sequence of work represented by the grading, drainage and sedimentation, and erosion control plans as related to other major items of construction, beginning with the initiation of excavation and including the construction of any sediment basins or retention facilities or any other structural BMP’s.

Installation Maintenance And Repair Plan. The design and planning of all stormwater systems shall include detailed installation, maintenance, and repair procedures to ensure their continued performance. These plans will identify the parts or components of a stormwater system that needs to be maintained and the equipment and skills or training necessary. Provisions for the periodic review and evaluation of the effectiveness of the maintenance program and the need for revisions or additional maintenance procedures shall be included in the plan. A permanent elevation benchmark shall be identified in the plans to assist in the periodic inspection of the facility.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

5.16 Post Construction

5.16.010 As Built Plans

5.16.020 Landscaping And Stabilization Requirements

5.16.030 Inspection Of Stormwater Systems

5.16.040 Records Of Installation And Maintenance Activities

5.16.050 Failure To Meet Or Maintain Design Or Maintenance Standards

### 5.16.010 As Built Plans

All applicants are required to submit as-built plans for any structures located on-site after final construction is completed. The plan must show the final design specifications for all stormwater systems and must be sealed by a registered professional engineer licensed to practice in Utah. A final inspection by the City Public Works Director or designee is required before any performance security or performance bond will be released. The City Public Works Director or designee shall have the discretion to adopt provisions for a partial pro-rata release of the performance security or performance bond on the completion of various stages of development. In addition, occupation permits shall not be granted until corrections to all BMP’s have been made and accepted by the City Public Works Director or designee.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.16.020 Landscaping And Stabilization Requirements

Any area of land from which the natural vegetative cover has been either partially or wholly cleared by development activities shall be re-vegetated according to a schedule approved by the City Engineer. The following criteria shall apply to re-vegetation efforts:

- Reseeding must be done with an annual or perennial vegetative cover accompanied by placement of straw mulch or its equivalent of sufficient coverage to control erosion until such time as the vegetative cover is established over one hundred percent (100%) of the seeded area.

- Replanting with native woody and herbaceous vegetation must be accompanied by placement of straw mulch or its equivalent of sufficient coverage to control erosion until the plantings are established and are capable of controlling erosion.

- Any area of re-vegetation must exhibit a minimum of seventy percent (70%) of the vegetative cover throughout the year immediately following re-vegetation. Re-vegetation must be repeated in successive years until the minimum seventy percent (70%) density for one (1) year is achieved.

- In addition to the above requirements, a landscaping plan must be submitted with the final design describing the vegetative stabilization and management techniques to be used at a site after construction is completed. This plan will explain not only how the site will be stabilized after construction, but who will be responsible for the maintenance of vegetation at the site and what practices will be employed to ensure that adequate vegetative cover is preserved.

- Stabilization must occur as UTRC00000 in the case of a project that impacts greater than or equal to one acre or the UPDES Common Plan Permit UTRH00000 in the case of a project that impacts less than an acre that is part of a larger common plan of development or sale, which will collectively disturb greater than or equal to one acre.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.16.030 Inspection Of Stormwater Systems

Periodic inspections of facilities shall be performed as provided for in this Title and Section 4.2.4.2.4 of the Payson City Stormwater Management Program.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.16.040 Records Of Installation And Maintenance Activities

Parties responsible for the operation and maintenance of a stormwater system shall make records of the installation of the stormwater system and of all maintenance and repairs to the facility and shall retain the records for at least five (5) years. These records shall be made available to the City Engineer during inspection of the facility and at other reasonable times upon request.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.16.050 Failure To Meet Or Maintain Design Or Maintenance Standards

If a responsible party fails or refuses to meet the design or maintenance standards required for a stormwater system under this Title, the City Engineer, after reasonable notice, may correct a violation of the design standards or maintenance needs by

performing all necessary work to place the facility in proper working condition. In the event that the stormwater system becomes a danger to public safety or public health, the City Engineer shall notify in writing the party responsible for maintenance of the stormwater system. Upon receipt of that notice, the responsible person shall have fifteen (15) days to effect maintenance and repair of the facility in an approved manner. In the event that corrective action is not undertaken within that time, the City Engineer may take necessary corrective action. The cost of any action by the City Engineer under this section shall be charged to the party responsible.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

5.20 Waivers

5.20.010 General

5.20.020 Conditions For Waiver

5.20.030 Downstream Damage, Etc., Prohibited

5.20.040 Land Disturbance Permit Not To Be Issued Where Waiver Requested

### 5.20.010 General

Every applicant shall provide for post construction stormwater management as required by this Title, unless a written request is filed to waive this requirement. Requests to waive the stormwater management program requirements shall be submitted to the City Engineer for review, processing, and approval or forwarded to the City Council, where deemed appropriate by City Engineer, for approval.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.20.020 Conditions For Waiver

The minimum requirements for stormwater management may be waived in whole or in part upon written request of the applicant, provided that at least one of the following conditions applies:

- It can be demonstrated that the proposed development is not likely to impair attainment of the objectives of this Title.

- Alternative minimum requirements for on-site management of stormwater discharges have been established in a stormwater management program that has been approved by the City Engineer.

- Provisions are made to manage stormwater by an off-site facility. The off-site facility must be in place and designed to provide the level of stormwater control that is equal to or greater than that which would be afforded by on-site practices. Further, the facility must be operated and maintained by an entity that is legally obligated to continue the operation and maintenance of the facility.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.20.030 Downstream Damage, Etc., Prohibited

In order to receive a waiver, the applicant must demonstrate to the satisfaction of the City Engineer that the waiver will not lead to any of the following conditions downstream:

- Deterioration of existing culverts, bridges, dams, and other structures;

- Degradation of biological functions or habitat;

- Accelerated stream bank or streambed erosion or siltation;

- Increased threat of flood damage to public health, life, or property.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.20.040 Land Disturbance Permit Not To Be Issued Where Waiver Requested

No land disturbance permit shall be issued where a waiver has been requested until the waiver is granted. If no waiver is granted, the plans must be resubmitted with a stormwater pollution prevention plan.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

5.24 Existing Locations And Developments

5.24.010 Requirements For All Existing Locations And Developments

5.24.020 Requirements For Existing Problem Locations

5.24.030 Inspection Of Existing Facilities

5.24.040 Corrections Of Problems Subject To Appeal

### 5.24.010 Requirements For All Existing Locations And Developments

The following requirements shall apply to all locations and development at which land disturbing activities have occurred previous to the enactment of this Title:

- Denuded areas must be vegetated or covered under the standards and guidelines specified in the Development Guidelines and on a schedule acceptable to the City Engineer.

- Cuts and slopes must be properly covered with appropriate vegetation and/or retaining walls constructed.

- Drainage ways shall be properly covered in vegetation or secured with rip-rap, channel lining, etc., to prevent erosion.

- Trash, junk, rubbish, etc. shall be cleared from drainage ways.

- Stormwater runoff shall be controlled to the extent reasonable to prevent pollution of local waters. Such control measures shall include those methods and measures identified in the Payson City Stormwater Management Program. Such control measures may include, but are not limited to, the following:

  - Ponds.

    - Detention pond

    - Extended detention pond

    - Wet pond

  - Alternative Storage Measures.

    - Constructed wetlands

    - Infiltration systems

      - Infiltration/percolation trench

      - Infiltration basin

      - Retention basin

    - Drainage (recharge) well

    - Porous pavement

  - Filtering Systems.

    - Catch basin inserts/media filter

    - Sand filter

    - Filter/absorption bed

    - Filter and buffer strips

    - Open channel, Swale

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.24.020 Requirements For Existing Problem Locations

The City Engineer shall in writing notify the owners of existing locations and developments of specific drainage, erosion, or sediment problem affecting such locations and developments, and the specific actions required to correct those problems. The notice shall also specify a reasonable time for compliance.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.24.030 Inspection Of Existing Facilities

The City Engineer may, to the extent authorized by state and federal law, establish inspection programs to verify that all stormwater systems, including those built before as well as after the adoption of this Title, are functioning within design limits. These inspection programs may be established on any reasonable basis, including but not limited to: routine inspections; random inspections; inspections based upon complaints or other notice of possible violations; inspection of drainage basins or areas identified as higher than typical sources of sediment or other contaminants or pollutants; inspections of businesses or industries of a type associated with higher than usual discharges of contaminants or pollutants or with discharges of a type that are more likely than the typical discharge to cause violations of the municipality’s NPDES/UPDES stormwater permit; and joint inspections with other agencies inspecting under environmental or safety laws. Inspections may include but are not limited to: reviewing maintenance and repair records; sampling discharges, surface water, groundwater, and material or water in drainage control facilities; and evaluating the condition of drainage control facilities and other BMPs.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.24.040 Corrections Of Problems Subject To Appeal

Corrective measures imposed by the City Engineer under this Section are subject to appeal under PCC 5.44.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022

5.28 Inspections

5.28.010 Purpose

5.28.020 Scope

5.28.030 Access

5.28.040 Follow-Up Inspections

5.28.050 Record Keeping

### 5.28.010 Purpose

To be in accordance with the General Permit for Discharges for Small Municipal Separate Storm Sewer Systems (MS4), Permit No. UTR090000, the City will conduct inspections to monitor all discharges to natural water bodies including lakes, streams, canals, stormwater controls, and BMPs. Inspections shall be conducted in a manner consistent with Utah Code §19-5-108.3 and applicable Clean Water Act requirements.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.28.020 Scope

Inspections relating to the MS4 Permit include but are not limited to illicit discharges, construction activities and post construction operation and maintenance of stormwater controls, reviewing maintenance and repair records; sampling discharges, surface water, groundwater, and material or water in drainage control facilities; and evaluating the condition of drainage control facilities and other BMPs either publicly or privately owned. Inspection may include review of Stormwater Pollution Prevention Plans (SWPPPs), verification of Best Management Practices (BMPs), and documentation of compliance through physical or electronic means.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.28.030 Access

Visual Inspections. Visual inspections of discharges to natural water bodies, spills, stormwater related controls on private property within the city limits of Payson are permitted by the City Engineer at any time. The City may also utilize electronic site inspections, including geo-located and time stamped photographic documentation submitted by the permittee, to assess compliance with storm water requirements.

Electronic Inspections. In lieu of or in addition to in-person inspections, the City may require permittees to submit electronic inspection documentation sufficient to demonstrate compliance with approved SWPPP and BMP requirements. Electronic inspections shall not limit the City’s authority to conduct in-person inspections at any time.

Other Inspections. When a visual inspection is not adequate to determine the extent of discharges to natural water bodies, spills, or determine the status of stormwater related controls on private property, the City will give twenty-four (24) hours’ notice of the inspection to take place and the extent of the inspection. Equipment and manpower necessary to perform the inspection will be allowed to access and work as necessary to determine the state of the situation.

Emergency Inspections. During times of emergency including discharges to natural water bodies, spills, or potential damage to life or property, the City may access the location of concern as necessary and with the equipment required to determine the status of the situation. Reasonable attempts to contact the property owner prior to the inspection will be made prior to accessing private property.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.28.040 Follow-Up Inspections

During initial or routine inspections if problems are identified that require corrective actions, then a follow-up inspection will be scheduled. Follow-up inspections may be conducted in person or through electronic site inspection methods as determined by the City.

5.28.050 Record Keeping

A. The City may require permittees to maintain and submit records or inspections, BMP implementation, and corrective actions.

B. Records may include electronic inspection reports, photographs, logs, and other documentation sufficient to demonstrate compliance.

C. Such records shall be made available to the City upon request.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

5.32 Illicit Discharges

5.32.010 Scope

5.32.020 Prohibition Of Illicit Discharges

5.32.030 Prohibition Of Illicit Connections

5.32.040 Reduction Of Stormwater Pollutants By The Use Of Best Management Practices

5.32.050 Notification Of Spills

### 5.32.010 Scope

This Section shall apply to all water generated on developed or undeveloped land entering the municipal separate stormwater system. All violations identified under this Section shall be subject to the enforcement procedures set forth in Section 5.36.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.32.020 Prohibition Of Illicit Discharges

No person shall introduce or cause to be introduced into the municipal separate stormwater system any discharge that is not composed entirely of stormwater. The commencement, conduct, or continuance of any non-stormwater discharge to the municipal separate stormwater system is prohibited except as described herein:

Uncontaminated discharges from the following sources:

- Water line flushing or other potable water sources

- Landscape irrigation or lawn watering runoff with potable or secondary water

- Diverted stream flows

- Rising ground water

- Groundwater infiltration to storm drains

- Uncontaminated pumped groundwater

- Discharges from potable water sources

- Foundation or footing drains

- Crawl space pumps

- Individual residential car washing

- Air conditioning condensation

- Irrigation water

- Springs

- Natural riparian habitat or wetland flows

- Water reservoir discharge ( if de-chlorinated – typically less than one PPM chlorine)

- Residual street wash water

- Firefighting activities

- Any other uncontaminated water source

- Discharges specified in writing by the City Engineer as being necessary to protect public health and safety

- Dye testing is an allowable discharge if the City Engineer has so specified in writing

- The prohibition shall not apply to any non-stormwater discharge permitted under an UPDES permit, waiver, or waste discharge order issued to the discharger and administered under the authority of the State of Utah Division of Water Quality, provided that the discharger is in full compliance with all requirements of the permit, waiver, or order and other applicable laws and regulations, and provided that written approval has been granted for any discharge to the stormwater system.

Violation Categories:

Applicable fines and penalties shall be imposed in accordance with Section 5.36.

- Construction-Related Violation. Violations associated with construction stormwater activity, including failure to implement or maintain Best Management Practices (BMPs) or comply with an approved Stormwater Pollution Prevention Plan (SWPPP), shall be enforced in accordance with Section 5.36 and Utah Code §19-5-108.3. Such violations shall be subject to progressive enforcement, including written warnings prior to the imposition of administrative fines, except in cases of immediate or imminent threat to water quality, public safety, or the environment.

- Improper Discharge to the City Sewer System. $500 per occurrence, per day of violation.

- Improper Discharge to the City Storm System. $500 per occurrence per day of violation.

- Improper Discharge to Water/Way. $500 per occurrence per day of violation.

- Immediate Enforcement Authority. Notwithstanding the foregoing, the City may impose immediate enforcement actions, including administrative fines or stop work orders, where the City determines that a discharge:

- Poses an immediate threat to water quality;

- Presents an imminent threat of discharge within a short timeframe;

- Creates a risk to public health, safety, or the environment; or

- Involves the unlawful discharge of pollutants.

In the event of any conflict between this Section and Section 5.36 regarding enforcement procedures or penalties, the provisions of Section 5.36 shall govern.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.32.030 Prohibition Of Illicit Connections

- The construction, use, maintenance, or continued existence of illicit connections to the city's stormwater system is prohibited.

- This prohibition expressly includes, without limitation, illicit connections made in the past, regardless of whether the connection was permissible under law or practices applicable or prevailing at the time of connection.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.32.040 Reduction Of Stormwater Pollutants By The Use Of Best Management Practices

Any person responsible for a property or premises, which is, or may be, the source of an illicit discharge, may be required to implement, at the person's expense, the BMP’s necessary to prevent the further discharge of pollutants to the municipal separate stormwater system. Compliance with all terms and conditions of a valid NPDES permit authorizing the discharge of stormwater associated with industrial activity, to the extent practicable, shall be deemed compliant with the provisions of this Section.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.32.050 Notification Of Spills

Notwithstanding other requirements of law, as soon as any person responsible for a facility or operation, or responsible for emergency response for a facility or operation has information of any known or suspected release of materials that are resulting in, or may result in, illicit discharges or pollutants discharging into stormwater system, the person shall take all necessary steps to ensure the discovery, containment, and cleanup of such release.

In the event of such a release of hazardous materials, the person shall immediately notify emergency response agencies of the occurrence via emergency dispatch services. In the event of a release of non-hazardous materials, the person shall notify the Stormwater Superintendent in person or by telephone no later than the next business day. Notifications in person or by telephone shall be confirmed by written notice addressed and mailed to the Stormwater Superintendent within three (3) business days of the telephone notice.

If the discharge of prohibited materials emanates from a commercial or industrial establishment, the owner or operator of such establishment shall also retain an on-site written record of the discharge and the actions taken to prevent its recurrence. Such records shall be retained for at least five (5) years.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

5.36 Enforcement

5.36.010 Enforcement Authority

5.36.020 Notification Of Violation

5.36.030 Conflicting Standards

### 5.36.010 Enforcement Authority

### The Storm Water Utility may adopt procedures and rules for the implementation and administration of this Chapter and to provide for the enforcement of the provisions contained herein.

### Authorized code enforcement personnel working under the direction of the City Manager are hereby declared to be public officials within the meaning of Section 77-7-18 Utah Code as amended, and these public officials are hereby authorized to issue misdemeanor and/or administrative citations and/or notices of violations, and/or stop work orders for violations of this Chapter or City Construction Standards and Specifications.

### Whenever the City finds that a person has violated a prohibition or failed to comply with any requirement(s) of this Chapter, the City may, as allowed by law; order compliance through a written notice of violation, administrative citation to the responsible person(s), or criminal prosecution. Administrative citation or notices of violation may require without limitation:

### The performance of monitoring, analyses, and reporting.

### The elimination of illicit connections or discharges;

### That violating discharges, practices, or operations shall cease and desist;

### The abatement or remediation of storm water pollution or contamination hazards and the restoration of any affected property;

### Payment to cover administrative, remediation, monitoring, analyses, and reporting costs; or

### The implementation of source control or treatment BMPs.

The City Engineer shall have the authority to issue notices of violation, stop work orders, fines, and citations, and to impose the civil penalties provided in this Section.

With the issuance of a Land Disturbance Permit, SWPPP and Operator Electronic Opt-out Waiver, the City MS4 Coordinator or RSI shall be permitted to enter and inspect, including testing and investigation, facilities subject to this Title at all reasonable times and as often as necessary to determine compliance. Failure to comply with the terms of this Title may result in punitive actions by Payson City Ordinance Enforcement, by Utah County Health Department, or by other means identified in permits or terms set forth in development applications. This Section shall govern all enforcement actions under this Title, including violations identified in Section 5.32.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 08-17-2022-D on 8/17/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.36.020 Notification Of Violation

- Written Notice. Whenever the Stormwater Superintendent finds that any permittee or any other person discharging stormwater is violating this Title or a permit or order issued hereunder, the Stormwater Superintendent may serve

- Notice of Violation (NOV). A NOV  may be issued when the City determines that a person has violated or failed to comply with any requirement of this Chapter including any requirement contained in a  permit or approved plan.

- Issuance and Enforcement of Construction Related NOV:

- Contents. The NOV shall state the location and nature of the noncompliance and shall also specify what action is required for the person to avoid revocation or suspension of any permit or plan issued pursuant to this Chapter. The NOV shall be sent to the responsible person in any permit, plan, or application or to the landowner.

- Process. The City of Payson follows the NOV process described in Utah State Code 19-5-108.3. This process applies to all construction-related violations under this Title, including those identified in Section 5.32.

- Exceptional Circumstances. The City may issue an order to stop construction earlier than described if the City has a clearly documented reason articulating an immediate threat to water quality, situations which involve a risk of injury to persons, damage to storm water facilities, or damage to other property or the environment.

- The City may recoup the costs incurred to correct a violation the person responsible refuses to correct after the enforcement process described by state law has been exhausted.

- Issuance and Enforcement of Other NOVs:

- Contents. The NOV shall state the location and nature of the noncompliance and shall also specify what action is required for the person to avoid escalating enforcement measures. The NOV shall be sent to the responsible person or to the landowner.

- Corrective Action. The NOV shall allow the person a reasonable time to take the necessary corrective action to avoid citations or other enforcement measures and shall be given, in the absence of exceptional circumstances, no less than ten (10) nor more than thirty (30) days to correct the violation.

- Exceptional Circumstances. For purposes of this Section, exceptional circumstances include, but are not limited to, an immediate threat to water quality, situations which involve a risk of injury to persons, damage to storm water facilities, or damage to other property or the environment. The City may take any steps the City deems necessary to alleviate any such exceptional circumstances as defined above, and may bill the landowner, developer, or contractor responsible for creating the exceptional circumstances for the cost of alleviating said circumstances.

- NOV Appeals. The person receiving the NOV may appeal any decision or directive made by the City. The party desiring to appeal shall file a notice of appeal at the City Public Works Directors Office within 10 days of the decision or directive being appealed. The notice of appeal shall contain the following information:

- The appellant’s name, address and daytime telephone number;

- A short statement describing the basis for the appeal; and

- The relief sought by the appellant.

- Hearing. Upon receipt of the notice of appeal, the City Public Works Director shall set a date for an informal hearing to consider the appeal. The informal hearing shall be conducted in accordance with policies established by the City Public Works Director. The City Public Works Director shall uphold the decision or directive being appealed unless the City Public Works Director finds that there has been an error in the interpretation or implementation of this ordinance. The City Public Works Director shall render a decision on the appeal within 10 days of the informal hearing with the appellant. The City Public Works Director shall have authority to affirm, reverse, or modify any decision or directive appealed pursuant to this section.

5.36.030 Administrative Citation.

- When a City Public Works Officer determines that a violation of this Chapter exists, the City Public Works Officer may issue an administrative citation to the responsible person not in conflict with state law.

- Content of Administrative Citation. The administrative citation shall include the following:

- The name and address of the responsible person;

- The street address of the violation or a description sufficient for identifying the building, structure, premises, or land upon or within which the violation is occurring;

- A description of the violation and the City

- Code section violated;

- An order prohibiting the continuation or repeated occurrence of the code violation described on the Administrative Citation;

- An order to the person responsible to correct the violation(s) within the time specified, and an explanation of the consequences of failure to correct the violation(s), including the fine for the violation;

- The amount of the fine and/or penalty and interest for the violation(s);

- An explanation of how the fine shall be paid and the time period by which it shall be paid;

- The time for appealing the administrative citation to the Hearing Officer and the procedure for filing an appeal; and

- A statement that the City may abate the violation and assess costs and expenses of abatement and a monetary fine against the person responsible if the correction is not completed by the responsible Person and approved by the City Public Works Director before the Completion Date.

- Service of Administrative Citation. The City Public Works Officer shall serve the administrative citation upon the person responsible, either personally or by mailing, certified, return receipt requested, a copy of the administrative citation to the person responsible at the last known address. If the responsible person cannot after due diligence be personally served within Utah County and if an address for mailed service cannot be ascertained after due diligence, notice shall be served by posting a copy of the administrative citation conspicuously on the affected property or structure. Proof of service shall be made by a written declaration under penalty of perjury executed by the person effecting the service, declaring the time and date of service, the manner by which the service was made, and if by posting, the facts showing that due diligence was used in attempting to serve the person personally or by mail.

- Fine Amounts. The amounts of the fines imposed for violations of this Chapter shall be set forth in the regular schedule of fines established by a Resolution of the City Council

If an applicant does not correct the specific violation for which the applicant received notice within the timeline set, the authority may impose an administrative fine for each occurrence as follows:

- $500 per occurrence for working without an approved storm water permit;

- $300 per occurrence for tracking mud on road;

- $250 per occurrence for failure to clean up or report spills;

- $100 per occurrence for failure to conduct storm water inspections;

- $100 per occurrence for failure to maintain storm water records; and

- $500 per site, per occurrence, for failure to use general best management practices, as determined by the authority; and may impose the administrative fine for each business day the specific violation continues beginning on the day after the day on which the authority issues the administrative fine; and within 30 days after the day on which the applicant corrects the violation.

- Improper Discharge to the City Sewer System. $500 per occurrence/day not to exceed $300if multiple days are in violation.

- Improper Discharge to the City Storm System. $500 per occurrence/day not to exceed $300if multiple days are in violation.

- Improper Discharge to Water/Way. $500 per occurrence/day not to exceed $3,000 if multiple days are in violation.

- Failure to Pay Fines. The failure of any person to pay the civil fines assessed by an Administrative Citation within the time specified on the citation or after an administrative hearing, will result in the collection of the fine by the City. The City may pursue any available legal remedy to collect civil fines, including but not limited to judgments, liens, small claims actions, and collections. The City may also recover its collections costs according to proof.

- Stop Work Order. Upon the occurrence of a third violation the City may issue a Stop Work Order in conjunction with the Administrative Citation.

- No Extension. No extension of the time specified in the administrative citation for correction of the violation may be granted, except by order of the Hearing Officer.

- Appeals.

- Grounds. Any person receiving an administrative citation may appeal the administrative citation to the Hearing Officer. Only the following issues may be appealed to the Hearing Officer:

- The person charged in the administrative citation as the responsible person, is not the responsible person as defined by this ordinance.

- The condition described as a violation in the administrative citation is not a violation as defined by this ordinance.

- The responsible person claims that the requirement(s) of the administrative citation violates constitutional rights.

- 5.36.40. Criminal Prosecution

- Violations of this Chapter are class C misdemeanors.

- The City may immediately proceed with criminal action against the violator if (1) the violator has committed the same violation in the past, or (2) the violation, in the opinion of the City, creates a serious risk and/or hazard to persons, the environment or property, or (3) the City deems the violation to constitute an emergency.

- 5.36.50 Nonexclusive Remedies. The City may take any or all of the abovementioned remedies (administrative, civil, or criminal) to abate a violation and/or to punish any person or entity that creates, causes, or allows a violation to exist. The abatement of a violation does not prejudice the right of the City or any person to recover damages or penalties for its past existence.

upon such person written notice of the violation. Within ten (10) days of this notice, an explanation of the violation and a plan for the satisfactory correction and prevention thereof, to include specific required actions, shall be submitted to the Stormwater Superintendent. Submission of this plan in no way relieves the discharger of liability for any violations occurring before or after receipt of the notice of violation.

- Consent Orders. The City Engineer is empowered to enter into consent orders, assurances of voluntary compliance, or other similar documents establishing an agreement with the person responsible for the noncompliance. Such orders will include specific action to be taken by the person to correct the noncompliance within a time period also specified by the order. Consent orders shall have the same force and effect as administrative orders issued pursuant to paragraphs D and E below.

- Show Cause Hearing. The City Engineer may order any person who violates this Title or permit, or order issued hereunder, to show cause why a proposed enforcement action should not be taken. Notice shall be served on the person specifying the time and place for the meeting, the proposed enforcement action, and the reasons for such action, and a request that the violator show cause why this proposed enforcement action should not be taken. The notice of the meeting shall be served personally or by registered or certified mail (return receipt requested) at least ten (10) days prior to the hearing.

- Compliance Order. When the City Engineer finds that any person has violated or continues to violate this Title or a permit or order issued thereunder, the City Engineer may issue an order to the violator directing that, following a specific time period, adequate structures, devices, be installed or procedures implemented and properly operated. Orders may also contain such other requirements as might be reasonably necessary and appropriate to address the noncompliance, including the construction of appropriate structures, installation of devices, self-monitoring, and management practices.

- Cease and Desist Orders. When the Stormwater Superintendent finds that any person has violated or continues to violate this Title or any permit or order issued hereunder, the Stormwater Superintendent may issue an order to cease and desist all such violations and direct those persons in noncompliance to:

- Comply forthwith; or

- Take such appropriate remedial or preventive action as may be needed to properly address a continuing or threatened violation, including halting operations and terminating the discharge.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.36.030 Conflicting Standards

Whenever there is a conflict between any standard contained in this Title and any referenced development guidelines or manual, the applicable standard shall be interpreted an applied in a manner consistent with Utah Code §19-5-108.3 and shall not exceed federal Clean Water Act requirements unless expressly authorized by state statute.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

5.40 Penalties

5.40.010 Violation

5.40.020 Measuring Civil Penalties

5.40.030 Recovery Of Damages And Costs

5.40.040 Other Remedies

5.40.050 Remedies Cumulative

### 5.40.010 Violation

In accordance with Utah Code § 10-9a-803 (1953 as amended), any applicant that violates this Title may be charged with a Class C misdemeanor or an appropriate civil penalty and subject to all fines and imprisonment associated with such penalty per day for each day of violation. Each day of violation shall constitute a separate violation.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022

### 5.40.020 Measuring Civil Penalties

In assessing a civil penalty, the City Engineer may consider:

- The harm done to the public health or the environment;

- Whether the civil penalty imposed will be a substantial economic deterrent to the illegal activity;

- The economic benefit gained by the violator;

- The amount of effort put forth by the violator to remedy this violation;

- Any unusual or extraordinary enforcement costs incurred by the City;

- The amount of penalty established by ordinance or resolution for specific categories of violations; and

- Any equities of the situation that outweigh the benefit of imposing any penalty or damage assessment.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.40.030 Recovery Of Damages And Costs

In addition to the civil penalty in PCC 5.40.020, the City may recover;

- All damages proximately caused by the violator to the City, which may include any reasonable expenses incurred in investigating violations of, and enforcing compliance with, this Title, or any other actual damages caused by the violation.

- The costs of the city’s maintenance of stormwater systems when the user of such facilities fails to maintain them as required by this Title.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.40.040 Other Remedies

The City may bring legal action to enjoin the continuing violation of this Title, and the existence of any other remedy, at law or equity, shall be no defense to any such actions.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.40.050 Remedies Cumulative

The remedies set forth in this Section shall be cumulative, not exclusive, and it shall not be a defense to any action, civil or criminal, that one (1) or more of the remedies set forth herein has been sought or granted.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022

5.44 Appeals

5.44.010 Appeals To City Council

5.44.020 Appeals To Be In Writing

5.44.030 Public Hearing

5.44.040 Appealing Decisions Of The Municipality's Governing Body

### 5.44.010 Appeals To City Council

Pursuant to Utah Code § 17-27a-707, any person aggrieved by the imposition of a civil penalty or damage assessment as provided by this Title may appeal said penalty or damage assessment to the City Council.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.44.020 Appeals To Be In Writing

The appeal shall be in writing and filed with the city recorder within fifteen (15) days after the civil penalty and/or damage assessment is served in any manner authorized by law.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.44.030 Public Hearing

Upon receipt of an appeal, the City Council shall hold a public hearing within thirty (30) days. Ten (10) days prior notice of the time, date, and location of said hearing shall be published as a Class A Notice according to Utah Code § 63G-30-102 (1953 as amended). Ten (10) day notice by registered mail shall also be provided to the aggrieved party, such notice to be sent to the address provided by the aggrieved party at the time of appeal. The decision of the City Council shall be final.

# HISTORY

## Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.44.040 Appealing Decisions Of The Municipality's Governing Body

Any alleged violator may appeal a decision of the City Council pursuant to the provisions of Utah Code § 17-27a-707 (1953 as amended).

# HISTORY

Adopted by Ord. 05-18-2022-D on 5/18/2022 Amended by Ord. 11-15-2023-A on 11/15/2023

### 5.44.050 Construction Stormwater Runoff Control

### Purpose.

### This section is adopted to comply with Utah Code § 19-5-108.3 and applicable provisions of the federal Clean Water Act governing construction stormwater runoff and municipal separate storm sewer systems (MS4).

### B. Applicability.

### This section applies to any person conducting construction activities resulting in a land disturbance of one acre or more, or less than one acre if part of a common plan of development or sale requiring permit coverage.

### C. Compliance with Federal Law.

### (1) The City shall not adopt, implement, or enforce any requirement under this chapter that is more stringent than the minimum requirements of the federal Clean Water Act, unless expressly authorized by state statute.

### (2) The City shall administer this chapter consistent with Utah Code § 19-5-108.3.

### D. Best Management Practices (BMPs).

### (1) The City shall determine and maintain a list of preferred Best Management Practices (BMPs) applicable to construction activities.

### (2) The BMPs shall comply with applicable federal standards and regulations.

### (3) The City shall publish and maintain the BMP list on the City’s official website.

### (4) All permittees shall implement BMPs in accordance with an approved stormwater pollution prevention plan (SWPPP).

### E. Permit Application Requirements.

### (1) The City shall maintain a list of requirements necessary for a complete construction stormwater permit application.

### (2) The City shall publish and maintain such list on the City’s official website.

### (3) An application shall not be deemed complete unless all required materials are submitted.

### F. Stormwater Pollution Prevention Plan Review.

### (1) The City shall review a complete stormwater pollution prevention plan within fourteen (14) business days after submission.

### (2) If the plan is incomplete, the City shall notify the applicant of deficiencies.

### (3) Construction activity requiring permit coverage shall not proceed until the SWPPP is approved or authorized.

### G. Inspections.

### (1) The City may conduct oversight inspections of construction sites to ensure compliance.

### (2) The City may allow or require electronic site inspections consisting of geo-located and time-stamped photographic documentation submitted by the applicant or permittee.

### (3) Participation in electronic inspections does not relieve compliance obligations.

### H. Violations.

### A violation includes failure to implement or maintain approved BMPs or comply with an approved SWPPP.

### I. Recordkeeping.

### Permittees shall maintain records of inspections, BMP implementation, and corrective actions and shall provide such records to the City upon request.