REQUEST FOR SEALED BIDS

Millard County Road Department is accepting sealed bids for a 36” x 150’ Telestacker Conveyer Belt with the following minimum specifications:

- 460V Motors

- Drive Class I head end

- Drive Pulley: 18” diameter, 3/8” herringbone lagged drum

- Tail Pulley:  16” diameter, CEMA Chevron wing pulley

- Shafts:  turned and polished

- Bearings:  Dodge

- Conveyor splice:  hydraulic top folding head section, with integral belt keeper

- Undercarriage:  hydraulic raise, with 7.5 hp electric pumping unit

- Radial travel:  1 axle, 2 hp electric

- Fifth wheel:  for road travel, bolt on design

- Anchor pivot:  maintains tail end during radial travel

- Air brake system

- Brake and directional signals

- Mudflaps

- Manual pin adjustment

- Pintle type hitch

- Wireless remote control

- Superior Idlers: CEMA B

- Self-training idlers

- Belting: 2-ply 3/16” x 1/16” 220 PIW

- Flexco belt scrapers or equivalent

- Overtravel limit switch

- 160 feet of Type – W power cable

- Manual control system

- Analog hour meter

Bids must include all fees and delivery to Delta, Utah.

Any questions regarding specifications can be directed to Brandon Winget 435-864-2467.

Please remit sealed bids to Millard County Auditors Office at 50 South Main, Fillmore, Utah 84631 or email to millardbids@co.millard.ut.us.  Please note “Stacker Belt” on envelope or subject line of email.  Bids must be received by 5:00 pm, July 6, 2026. Bids will be opened at the Millard County Commission meeting on July 7, 2026.

Millard County reserves the right to accept or reject any or all bids.