13.40 Minor Subdivision Requirements And Procedures

13.40.010 Purpose
13.40.020 Procedures

13.40.030 Flowchart Table
13.40.040 Sketch ConceptPlan
13.40.050 Record Survey
13.40.060 Construction Drawings
13.40.070 Final Plat
13.40.080 Performance Bond And Fees

13.40.010 Purpose

The City recognizes based on current ordinances, that many smaller parcels of land exist within the residential areas of Hyde Park City that have adequate road frontage and acreage to potentially be split into additional building lots. This section allows the subdivision of smaller parcels of land into four (4) building lots or less, with the existing residence to be located on one (1) of the lots.

HISTORY
Adopted by Ord. 2015-03 on 5/13/2015

13.40.020 Procedures

This section provides the procedures required for a minor subdivision by Hyde Park City, for submitting a sketch concept plan, preparing a “record survey”, and preparing construction drawings. THE ENTIRE PARCEL WITHIN THE SAME OWNERSHIP MUST BE INCLUDED IN THE MINOR SUBDIVISION.

The intent of a Minor Subdivision is to streamline the approval process only, all required subdivision improvements, as outlined in all the other chapters of the Subdivision Ordinance, will be followed and constructed as outlined. Any changes requested by the developer that do not conform to the ordinance can only be approved by the City Council.

HISTORY
Adopted by Ord. 2015-03 on 5/13/2015

13.40.030 Flowchart Table

13.40.040 Sketch Concept Plan

The sub divider shall digitally submit six (6) a copiesy of a sketch concept plan, at least 11” x 17”, to the LUA for review. and discussion on the site plan showing general lot layout and frontages.  Plans shall be drawn to scale showing all lot dimensions being requested.  Copies shall be submitted five (5) working days prior to scheduling a meeting with the LUA, and copies not submitted beforehand shall not be considered until the LUA has had a minimum of seven (7) working days to consider the plan.

The plan shall include the following items:

- General lot layout and frontages.

- The property boundaries of the proposed new lots.

- Names of adjacent property owners.

- Number of lots proposed and street frontage for each.

- Total acreage of each lot based on present zoning.

- Approximate location of all existing utilities that will service the lots.

The LUA will review the sketch concept plan with the developer and determine if it meets the requirements of a Minor Subdivision in accordance with the ordinance.  The LUA will also review all the required infrastructure improvements that will need to be constructed by the developer in order to proceed with the subdivision.

HISTORY
Adopted by Ord. 2015-03 on 5/13/2015

13.40.050 Record Survey

Following the review, if the sketch concept plan is approved, the developer shall have a “record survey” prepared by a licensed engineer. The record survey shall clearly show the division of the proposed lots and shall include all dimensions, square footages, and metes and bounds descriptions. The record survey shall be prepared in accordance with the standards established by the county surveyor and include a signature line for the Planning Commissioner and the City Attorney to sign for acceptance and approval of the record survey.

HISTORY
Adopted by Ord. 2015-03 on 5/13/2015

13.40.060 Construction Drawings

The developer shall prepare construction drawing showing all improvements required by the subdivision ordinance, including but not limited to:

- water and sewer connections

- curb and gutter

- sidewalk

- road extensions

- storm water control

- secondary water extensions and improvements

Construction drawings also need to contain a place for the LUA to indicate its approval and acceptance of the record survey.

The construction drawings will be reviewed and approved by the DRC.

HISTORY
Adopted by Ord. 2015-03 on 5/13/2015

13.40.070 Final Plat

Once all the above requirements are presented and approved by the DRC they shall be forwarded to the Planning and Zoning Commission for final approval. All adjacent property owners will be notified of the meeting when Planning & Zoning Commission reviews the proposed minor subdivision. Based on the approval of the Planning Commission the developer shall have their engineer file the record survey and have prepared those deeds required to have the new lots recorded with the County Recorder by Hyde Park City. Said deeds shall include the words “approved and accepted by Hyde Park City” with a signature line for a designated representative of Hyde Park City to sign.

HISTORY
Adopted by Ord. 2015-03 on 5/13/2015
Amended by Ord. 2019-05 on 4/24/2019

13.40.080 Performance Bond And Fees

No construction improvements shall begin on the Minor Subdivision until the developer has furnished the city with a form of security, as outlined in HPMC 13.50, to insure the proper and prompt installation of all improvements as required.

Any and all persons requesting a Minor Subdivision shall first have paid all fees set forth by the most recent prevailing fee schedule adopted by resolution of the Hyde Park City council. The approved Minor Subdivision will require the payment of inspection fees as also outlined by the city prior to any work beginning.

HISTORY
Adopted by Ord. 2015-03 on 5/13/2015