PUBLIC NOTICE

Invitation to Bid

Milford City is accepting SEALED BID from general contractor for a Swimming Pool resurfacing and repair project at the Milford City Pool located at 140 North 225 West, Milford Utah:

Project: Swimming Pool Resurfacing and Repairs

- Replaster the lap and kiddie pool

  - Undercut tile edges

  - Completely remove all existing plaster down to the gunite shell and properly dispose of the construction materials into a roll-off dumpster

  - Completely replaster lap and kiddie pool.  Each batch should contain the following:

    - (4)  white cement

    - (4)  white sand

    - (4)  white quartz

  - Pool Size:  Lap Pool-Approximately 3306 sq ft

Kiddie Pool- Approximately 374 sq ft

  - Install new drain covers and wall fittings as needed

  - Repair loose drainage deck

  - Add quartz in the finished plaster

  - Replace the broken pool edging on the south side of the kiddie pool

  - Install new tile as needed around the upper cap of the pools.  New tile needs to be installed along the 5’ markings to show the deep end (same location as the rope).  Swim lap tiles are not needed for swim lanes.  The color of the tile shall be a dark contrasting color or substantially similar to existing tile color. All tile dimensions shall comply with state and federal requirements.

  - Reattach or replace ladders and hand rails

  - Work to be completed by April 30, 2022 to allow opening the end of May 2022

  - Provide a three-year warranty with applicable terms

City Responsibility

  - Milford City shall drain all water from both of the pools

  - City will provide a roll-off dumpster and forklift as needed

    - Roll-off dumpster should not be any more than 1/3 full, due to the heavy material being disposed of.

Bids must be honorable for 60 days.

On site walk through is available by contacting Pool Manager, Melissa Rose at 435.421.2051.  Questions regarding the project may also be directed to her. If you are unable to get in contact with the pool manager, please call the City Administrator, Makayla Bealer at 435.387.2717.

BID SUBMITTAL DEADLINE:  4:00 PM, Thursday, October 21, 2021

BIDS MAY BE SUBMITED AT THE FOLLOWING LOCATIONS:

Submit by person:  Milford City Office, 26 South 100 West, Milford UT 8:00 AM to 4:00 PM

Mail to: Milford City, PO BOX 69, Milford, UT 84751 – Clearly Mark BID on envelope

Emailed to: Makayla Bealer at mbealer@milford.utah.gov – Clearly Mark BID in Subject Line

BID OPENING: Friday, October 22, 2021 at 10:00 AM, Milford City Office, 26 South 100 West Milford, Utah.