D I S C U S S I O N   P A P E R  F O R   P L A N N I N G   C O M M I S S I O N   U S E

TITLE 10

ZONING REGULATIONS

Red = Changes required by Utah law

Blue = Changes to be discussed by Planning Commission

CHAPTER 5
A-1 AGRICULTURAL ZONE

SECTION:

10-5-1: Regulations

10-5-2: Intent

10-5-3: Permitted Uses

10-5-4: Area And Frontage Requirements

10-5-5: Location Requirements

10-5-6: Height Of Buildings

10-5-7: Size Of Dwelling

10-5-8: Width Of Buildings

10-5-9: Yard Space For One Main Building Only

10-5-10: Every Dwelling To Be On A Lot or Parcel

10-5-11: Sale Or Lease Of Required Space Prohibited

10-5-12: Yards To Be Unobstructed; Exceptions

10-5-13: Accessory Buildings Prohibited As Living Quarters

10-5-14: Storage Of Junk, Debris, Or Unlicensed Inoperable Vehicles Prohibited

10-5-15: Sewage Disposal

10-5-16: Water Connections

10-5-17: Drainage

10-5-18: Clear View Of Intersecting Streets

10-5-19: Effect Of Street Plan

10-5-20: Motor Vehicle Access

10-5-21: Landscaping Required

10-5-22: Pollution Prevention

10-5-23: Special Provisions

10-5-1: REGULATIONS:

The use, location, height and size of buildings and structures, the use of land and size of lots, yards, courts and other open spaces and the density of population are regulated as hereinafter set forth. (Ord. 2012-01, 7-10-2012)

10-5-2: INTENT:

A-1 agricultural zone covers certain fringe areas of the city where residential areas may be integrated with the raising of livestock for family food production and for the pleasure of the people who reside on the premises. (Ord. 2012-01, 7-10-2012)

10-5-3: PERMITTED USES:

- The following buildings, structures and uses of land shall be permitted upon compliance with requirements set forth in this title:

Accessory buildings.

Accessory uses and structures shall be permitted in the C-2 zone, provided they are incidental to and do not substantially alter the character of the permitted principal use or structure.

Bunkhouses used by workers and staff.

Churches, not including temporary revival tents or buildings.

Fences, walls and hedges.

Foster care homes.

Grain bins and silos.

Household pets.

Landscape businesses, exclusive of gravel crushing and which do not violate dust, noise, or nuisance ordinances.

Machinery and equipment sheds used for storage of equipment in connection with agricultural activities performed on the premises.

Mobile home parks meeting the requirements of chapter 13 of this title.

Plant nursery.

Public agency arenas such as rodeo grounds, equestrian sports facilities, fairgrounds.

Public agency parks and playgrounds.

Public buildings and grounds, not including storage yards or repair shops.

Public, private and parochial schools and grounds.

Single- and two-family dwellings of conventional construction and manufactured homes.

The care and keeping of livestock and fowl.  Also barns, corrals, pens and sheds for the care and keeping of the permitted animals and fowl.

The growing of field crops and fruit.

Veterinary clinics and animal boarding facilities. (Ord. 2012-01, 7-10-2012)

- The following uses are permitted when in compliance with the specified sections of Title 10, Zoning Regulations:

All signs shall conform with section 10-2-6 of this title.

Conditional uses shall conform with section 10-2-8 of this title.

Temporary uses shall conform with section 10-2-10 of this title.

Portable Storage Containers shall conform with section 10-2-11 of this title

Fences shall conform with section 10-2-15 of this title.

- The following uses are permitted as conditional uses.

Daycare, nursery, preschool.

Rest homes, nursing homes, convalescent homes, assisted living homes.

10-5-4: AREA AND FRONTAGE REQUIREMENTS:

- Area and width requirements for all uses shall be a minimum of one acre.

- Frontage at setback line shall be a minimum of one hundred twenty five feet (125'). (Ord. 2012-01, 7-10-2012)

10-5-5: LOCATION REQUIREMENTS:

- Front Setback:

- Main Setback:  All dwellings and other structures shall be set back according to the special conditions noted below:

- Where the legal property line of the lot or parcel is located at the back of street curb or back of pavement and there is no curb, the front setback shall be fifty feet (50’) from the front lot or parcel line.

- Where the legal property line of the lot or parcel is located within the street pavement or gravel area, the front setback shall be forth feet (40’) from the front lot or parcel line.

- Where the legal property line of the lot or parcel is located more than ten feet (10’) from the street pavement or gravel area, the front setback shall be twenty feet (20’) from the front lot or parcel line.

- Where the legal property line of the lot or parcel line is angular to the roadway and there is less than twenty feet (20’) between the legal property line and the street pavement or gravel area, then the front setback shall be a minimum of twenty feet (20’) from the nearest point of the street pavement or gravel area.

- Accessory Buildings: All accessory buildings shall be set back not less than twenty feet (20') to the rear of the closest rear wall line of the main building and not less than twelve feet (12') from any side wall of the main building. Accessory buildings which are located twelve feet (12') or closer to a main building shall be considered as part of the main building. Where no main building exists on a lot or parcel, a detached accessory building shall be set back not less than sixty feet (60') from the front lot line.

- Side Setback; Interior Lot or Parcel:

- Main Buildings: All dwellings and other main buildings shall be set back at least twenty feet (20') from the side lot or parcel line.

- Accessory Buildings: All accessory buildings shall be set back at least twenty feet (20') from the lot or parcel line.

- Side Setback; Corner Lot or Parcel:

- Side Abutting Street: All dwellings and other main and accessory buildings shall be set back at least fifty feet (50') from the side lot or parcel line which abuts on a street.

- Interior Side: Same as required for interior lots or parcels.

- Rear Setback; Interior Lot or Parcel:

- Main Buildings: All dwellings or other main buildings shall be set back at least twenty feet (20') from the rear lot or parcel line.

- Accessory Building: Same as required under side setbacks for interior lots.

- Rear Setback; Corner Lot:

- Main Buildings: All dwellings and other main buildings shall be set back at least thirty feet (30') from the rear lot or parcel line, except

- Where a carport or garage is attached to the rear of the dwelling, the required rear setback for said carport or garage may be reduced to twenty feet (20') or more as measured from the rear lot or parcel line to the closest part of the building.

- Accessory Building: All accessory buildings shall be set back at least thirty feet (30') from the rear lot or parcel line

10-5-6: HEIGHT OF BUILDINGS:

The maximum height of any building shall be measured from finished grade at the front of the building to the highest wall plate.

- The maximum height of a dwelling building shall be as noted below:

- Level lot or parcel: Thirty feet (30').

- Uneven lot or parcel: Where the ground is uneven in height and the ground slopes downward from the front of the dwelling by more than ten feet (10’) to the back of the dwelling the maximum height shall be thirty-three (33').

B.	Minimum height of a dwelling building shall not be less than the height required by the applicable building code.

C.	The maximum height of accessory buildings shall be no greater than forty-five feet (45') or two stories, whichever is the higher.

D.	 Additional Height Allowed For Public Buildings:

Public buildings and churches may be erected to any height, provided the building is set back from required building setback lines a distance of at least one foot (1') for each additional foot of building height above the maximum height otherwise permitted in the zone in which the building is located. (Ord. 2012-01, 7-10-2012)

10-5-7: SIZE OF DWELLING:

The ground floor area of any single-family dwelling shall be at least nine hundred (900) square feet of living area exclusive of open porches, carports, garages and similar add ons. (Ord. 2012-01, 7-10-2012)

10-5-8: WIDTH OF BUILDINGS:

The total length or total width of any dwelling or church shall be at least fourteen feet (14'), as measured from outside wall to outside wall, exclusive of any carport, garage, shed or similar add on. (Ord. 2012-01, 7-10-2012)

10-5-9: YARD SPACE FOR ONE MAIN BUILDING ONLY:

No required yard or other open space around an existing building, or which is hereafter provided around any building, which is needed to comply with the provisions of this title, shall be considered as providing a yard or open space for any other building; nor shall any yard or other required open space on an adjoining lot or parcel be considered as providing the yard or open space on the lot or parcel whereon a building is to be erected or established. (Ord. 2012-01, 7-10-2012)

10-5-10: EVERY DWELLING TO BE ON A LOT OR PARCEL:

Only one building which contains a dwelling shall be located and maintained on a "zoning lot" lot or parcel as defined in this title, except bunkhouses and dwellings within a mobile home park or other large scale development. (Ord. 2012-01, 7-10-2012)

10-5-11: SALE OR LEASE OF REQUIRED SPACE PROHIBITED:

No space needed to meet the width, yard, area, coverage, parking, or other requirements of this title for a lot, parcel, or building may be sold or leased apart from such lot, parcel, or building unless other space so complying is provided. (Ord. 2012-01, 7-10-2012)

10-5-12: YARDS TO BE UNOBSTRUCTED; EXCEPTIONS:

Every part of a required yard shall be open to the sky and unobstructed except:

- Permitted accessory buildings, and

- Ordinary and customary projection of sills, belt courses, cornices and other ornamental features and unenclosed steps and unwalled stoops, porches and carports which may project up to three feet (3') into a required yard.

10-5-13: ACCESSORY BUILDINGS PROHIBITED AS LIVING QUARTERS:

Living and sleeping quarters in any building or structure other than the main residential building and bunkhouse are prohibited. (Ord. 2012-01, 7-10-2012)

10-5-14: STORAGE OF JUNK, DEBRIS, OR UNLICENSED INOPERABLE VEHICLES PROHIBITED:

- No yard or other open space surrounding an existing building, or which is hereafter provided around any building in any zone, shall be used for the storage of junk, debris, or inoperable vehicles;

- No land shall be used for such purposes, except as specifically provided by this title.

- Storage of one restorable vehicle may be permitted in the backyard for a period of six (6) months, on a case by case basis, at the discretion of the zoning administrator. Restoration work shall comply with applicable noise and nuisance ordinances.

10-5-15: SEWAGE DISPOSAL:

- Sewer connections shall comply with Title 8 of city code.

- Where domestic sewage disposal facilities are used which are not connected to a public sewer, approval of such facilities shall be obtained from the state health department before a building permit shall be issued therefor.  (Ord. 2012-01, 7-10-2012)

10-5-16: WATER CONNECTIONS:

- Where municipal water is available all dwellings shall be connected in accordance with Title 8 of city code.

- Where no municipal water is available, approval of such facilities shall be obtained from the state health department before a building permit or certificate of occupancy shall be issued therefore.

10-5-17: DRAINAGE:

- The owners and users of ditches shall not divert more water into the ditch than it's carrying capacity.

- Runoff from new rooftops or lots shall not exceed the carrying capacity of the natural or previous drainage system. (Ord. 2012-01, 7-10-2012)

- Drainage corridors shall be protected in accordance with 10-12-2.

10-5-18: CLEAR VIEW OF INTERSECTING STREETS:

No obstruction which will obscure the view of automobile drivers shall be placed on any corner lot within a triangular area formed by the street property lines and a line connecting them at points of forty five feet (45') from the intersection of the street lines. (Ord. 2012-01, 7-10-2012)

10-5-19: EFFECT OF STREET PLAN:

- Wherever a front or side yard is required for a building that abuts on a proposed street which has not been constructed, but which has been designated by the planning commission as a future street, the depth of such front or side yard shall be measured from the planned street lines. (Ord. 2012-01, 7-10-2012)

- Before a building permit shall be issued for the building that abuts on a proposed street, the property owner shall

- Grant an easement for the proposed street to the city.

- Construct the street and underground utilities to the standards required by the city.

- Post a bond guaranteeing that all street and underground utility improvements will be completed to standards required by the city.

10-5-20: MOTOR VEHICLE ACCESS:

Access to all lots and parcels of land having a frontage on a city street shall be controlled as follows:

- Access shall be by not No lot or parcel shall have more than two (2) driveways from any one street.

- Driveways shall not be closer to each other than twenty feet (20').  Where one driveway is shared by adjacent lots or parcels, a conditional use permit is required prior to constructing the driveway.

- On corner lots or parcels, no driveways shall be closer than fifteen feet (15') twenty feet (20’) to the point of intersection of the front property line with the side property line which abuts upon a street.

- Where there is no existing curb and gutter or sidewalk, a curb or fence may be required by the city. (Ord. 2012-01, 7-10-2012)

10-5-21: LANDSCAPING REQUIRED:

Front yards and side yards of all dwellings and bunkhouses which front on city streets must be landscaped. (Ord. 2012-01, 7-10-2012)

10-5-22: POLLUTION PREVENTION:

Any use which emits or discharges gases, fumes, dust, glare, noise or other pollutants into the atmosphere in amounts which exceed the standards as prescribed by Utah state air conservation board or the board of health, and any use which emits or discharges liquids or solid material onto the soil or water in amounts which result in pollutants entering groundwater in amounts exceeding the standards prescribed by the Utah state water pollution control board or the board of health, shall be prohibited. (Ord. 2012-01, 7-10-2012)

10-5-23: SPECIAL PROVISIONS:

Special provisions shall apply in order to protect the intent of the zone.

- All buildings used for human occupancy shall be furnished with a water supply in accordance with 10-5-15 of this title.

- All buildings used for human occupancy shall be constructed in accordance with the adopted building, plumbing, electrical, fire prevention and similar codes.

- All buildings and uses shall comply with all applicable supplementary development standards as set forth in this title. (Ord. 2012-01, 7-10-2012)